Waterloo TCP

Installation Notes

by Erick Engelke

Introduction          Introduction

TCP/IP is not a program, it is a set of protocols which have

been  implemented on many machines.  All machines running an

implementation  of TCP/IP  and connected  to the  world wide

Internet are capable of communicating with each other.

There    are    several   popular    non-commercial   TCP/IP

implementations for MS-DOS computers.   Each offers  special

features but with  varied drawbacks.  I  don't believe there

is a clear choice  of one implementation for all  needs, but

users  are free to pick the best or most useful applications

from each offering.

These  notes  describe  the various  applications  available

today.    Please remember  that  the  applications are  free

software, you  may use them and pass  them on to others, but

there is no warranty  and the support is very  limited.  You

also may not sell the included programs.

Installation          Installation

Waterloo  TCP  only works  if you  have  a packet  driver, a

special program which allows  your network interface card to

talk with the Waterloo TCP applications.

Thanks to  some very  generous people,  particularly Russell

Nelson, you probably will  not have to buy a  packet driver.

If you are  using Ethernet  hardware you  can probably  find

free  packet drivers  for  your cards  via anonymous  ftp to

sun.soe.clarkson.edu in the pub/drivers subdirectory.               ____________________        ___________

Waterloo  TCP only  supports  Class 1  drivers.   Officially

these  are called  Ethernet  class drivers,  but many  other

types  of  networks have  drivers  which  make them  emulate

Ethernet hardware.  For example, any Novell system using IPX

or any IBM compatible Token Ring  network can be made to act

like Ethernet.  Even serial lines (SLIP) emulate Ethernet if

you use  Michael Martineau's  ETHERSLIP driver  available by

anonymous ftp from owl.nstn.ns.ca in in.coming/etherslip.                                  ______________    ___________________

To start using Waterloo TCP software you will need to get it

configured.    There  are  two  options,  using BOOTP  or  a

configuration file.

If  you think  you may  have a  BOOTP server  on  your local

subnet, copy  the file  TCPINFO.EXE into a  new subdirectory

and  run the command  TCPINFO.  It  may take  a few seconds.

After a maximum of 30 seconds, TCPINFO should tell you if it

could get configured via  BOOTP.  If it could  not, or BOOTP

is too slow, you will have to use a configuration file.

You will  probably want a configuration file  anyways, as it

allows some  extra things which  are not inherent  in BOOTP.

Waterloo TCP lets  you use a config file,  and pick up extra

things from BOOTP.

If you don't  really know  what you are  doing, continue  on

with this section and make a config file.

First  you will  need  some important  information from  you

local TCP/IP guru.   Do not merely guess, these  values must

be correct or you may do some damage and get yourself on the

death threat list from your local network people.

IP address (eg. 4.1.2.23)

my_ip =  ______.______.______.______

local subnet mask (eg. 255.255.254.0, never 255.255.255.255)

netmask = ______.______.______.______

local gateway (eg. 4.1.2.1)

gateway = ______.______.______.______

primary name server (eg. 129.97.128.1)

nameserver = ______.______.______.______

alternate name servers (up to 9 more if so desired)

just keep repeating this line with new addresses.

nameserver = ______.______.______.______

name domains list, (eg. UWaterloo.ca  or  edu)

d   o   m   a   i   n   s   l   i   s   t                  =

__________________________________________________

These  values must be  placed in  a file  called WATTCP.CFG.

Below is a sample copy, remember, do  not use my values, get

the correct ones!

print="using sample configuration" # sample comment

print="contact local network guru for more details"

my_ip=129.97.176.99

netmask=255.255.254.0              # sample comment

nameserver=129.97.128.24           ; sample comment

nameserver=129.97.128.196          # alt nameserver

nameserver=129.97.128.1            # 3rd nameserver

gateway=129.97.176.1

domainslist="uwaterloo.ca"

The rules are simple, directive=value.

If quotes are not used in the value field, the value will be

terminated by  the start of a  comment or by a  newline, and

all white space (spaces and tabs) are removed.

If you specify quotes around the value, only a second set of

quotes  or a newline will  end the value  field and comments

must  be preceded  by  an end  quote  mark.   Whitespace  is

preserved inside quotes.

Place the  WATTCP.CFG file in  the same subdirectory  as the

TCP  application programs.  If  the file is  not found there

the programs automatically look for the file in the  current

subdirectory of the current  disk.  Failing that, a  message

will  be  displayed but  the  program  will not  necessarily

abort.

You may  override the above directory  choices by explicitly

setting the path in an environment variable.

eg.  set wattcp.cfg=n:\internet

The environment  variable is  checked first,  and  if it  is

defined  that   configuration  file   is  used.     This  is

particularly  useful on installations  where the software is

located on  a fileserver,  but individual  workstations will

need separate configuration files.

Testing          Testing

First,  to  ensure  that  you  entered  all  the  parameters

correctly, run  TCPINFO.  It will list all system constants.

If one or more  of them seem incorrect, check  your spelling

in the WATTCP.CFG file.

Next  we will test the  PING command to  see that everything

works  and asks  another computer  if it is  up.   The first

argument to  PING is  the name of  the other computer.   The

second argument is the  number of retries.  Since  your guru

supplied  the ip  address of  a gateway,  we will  first try

that.

ping  129.97.128.1  5         don't  use  129.97.128.1,

use  your   gateway's  IP

address

This will generate five attempts.  You should have more than

0  % success.   Otherwise  your gateway is  down or  your ip

address or gateway is wrong.

If  you  had success,  try pinging  the  ip address  of your

nameserver.

eg.  ping  129.97.128.196  5

Now check your nameserver by trying to resolve the name of a

local machine.  Near me is a machine named 'cupid'.

ping  cupid  5

If that did  not work, your  various nameserver entries  are

incorrect, your  gateway or network mask  is incorrect, your

nameservers did not want to provide name service, or you did

not specify a valid name.

These tests will  help your  guru figure out  what might  be

wrong.

Applications          Applications

TCPINFO               TCPINFO

Displays the current Ethernet/TCP configuration.  It is

useful for  testing spelling and contents  of files and

for determining ethernet addresses.

PING               PING

PING  [-d|/d]  [-s|/s]  hostname  [attempts]

You  have already  seen PING  described briefly  in the

installation section.  PING will not generate more than

one  request  per second,  it  also  attempts to  block

broadcast attempts.

PING can be used in a debugging mode (-d or /d).

eg.  PING  -d  129.97.128.1

If  you do  not specify  the number  of attempts  to be

made, only one attempt will be made.

eg.  PING  129.97.128.196

Specifying  /s will  ping  the other  machine once  per

second for a very long time.

eg.  PING  -s  129.97.128.196

COOKIE               COOKIE

COOKIE [host]

eg.  COOKIE

COOKIE  conehead.uwaterloo.ca

Print a witty saying from one of the cookie servers.

DAYTIME                DAYTIME

Print the time of day using TCP

DAYTIME  host

eg.  DAYTIME  129.97.128.1

DAYTIME  watmath.uwaterloo.ca

If the  host supports TCP based  DAYTIME text services,

the time of  day will  be displayed as  a text  string.

See also NTIME

FINGER               FINGER

Determine user or system information

FINGER [user]@host

eg.  FINGER  erick@sunee.uwaterloo.ca

FINGER  @sunee.uwaterloo.ca

Finger returns  the remote computer's information  on a

particular user.

If no user  is specified, FINGER will return  the names

of currently logged users on that machine.

LPR               LPR

Spool print jobs

LPQ               LPQ

Query the print queue

Run these commands with no arguments for the exact

syntax.   Check to  see that the  appropriate host

privileges are extended to the pc.

An explanation beyond this  is beyond the scope of

this brief document, see your local UNIX guru with

HOSTS.LPR   or   whatever   s/he   feels   is

appropriate.

NTIME               NTIME

Set DOS time from the Network.

NTIME  host  [addminutes]

NTIME contacts the host  and requests the current time.

Computers are  supposed to  respond with the  number of

seconds  since Jan 1, 1900 GMT.  Many simply return the

current time adjusted to  the daylight savings time and

time  zone.  I allow  you to specify  addminutes if you

need  to add or subtract a certain number of minutes to

the returned time.

I was considering using  a DST conversion algorithm but

have not yet done so.

TCPPORT                TCPPORT

Treat the serial port as a TCP connection

TCPPORT host port "program options"

Host is the name  or ip address of the  remote computer

and port is the TCP port number on that computer.

You  may  specify  the  terminal  emulation desired  by

setting the environment variable

set  tcpterm=termtype

eg.  set  tcpterm=vt102

See the section on TCPPORT below

REXEC               REXEC

Execute the following command on a remote host

REXEC  host  [user [pass]] cmd

The  "cmd"  command  will  be executed  on  the  remote

computer.  If  you fail to specify  either the password

or the userid, you will be prompted for them.

eg.  rexec  hq.iraq  "ls -l"

rexec  hq.iraq  saddam  "ls -l"

rexec  hq.iraq saddam white_flag_of_victory "ls"

REXEC does not do terminal interpretation, you may wish

to  have  NANSI.SYS  loaded  to  provide  the necessary

emulation.  Waterloo TCP REXEC is good when you wish to

redirect output to a file.

Other WATTCP Programs          Other WATTCP Programs

The above  programs are relatively  simple demonstrations of

the  capabilities of  the  WATTCP TCP/IP  kernal.   Advanced

programs are usually distributed  separately as they tend to

be  updated   in  a  different  schedule   from  the  kernal

libraries.

MSKERMIT 3.11               MSKERMIT 3.11

One  of  the first  popular  uses  for WATTCP  was  its

ability to make communication programs such as MSKERMIT

act like  TELNET facilities.   So overwhelming  was the

number of  requests that  MSKERMIT 3.11 now  includes a

derivative  of   the  WATTCP  kernal  and  the  TCPPORT

application.

TELNETD               TELNETD

The next  most popular use is easily TELNETD, a program

which  allows you to TELNET into your pc and control it

using  any  TELNET program  on  any  computer platform.

TELNETD   can   be   found   via   anonymous   ftp   to

sunee.uwaterloo.ca in pub/wattcp/telnetd.zip.

Using Communications Programs with TCPPORT          Using Communications Programs with TCPPORT

You may wish to use  a terminal communication program rather

than TELNET.  Waterloo TCP  makes this very easy to do  with

its  TCPPORT  program.    Now  that  TCPPORT  is built  into

MSKermit I don't really have a good example, but here goes:

Start by creating a configuration  file which tells your com

program  to use the BIOS  ports rather than  hardware.  Then

create a batch file which looks like:

TNCOMM.BAT

echo off

tcpport %1 23 "c:\comm"

Here I was assuming you kept comm.exe in the root  of C: and

tcpport could be found somewhere in  the path.  Now you  can

easily TELNET to any host by typing:

TNCOMM  host

eg.  TNCOMM  129.97.128.1

or   TNCOMM  watmath.uwaterloo.ca

After  you  log off,  Waterloo  TCP  returns the  characters

forming  [??Host  closed   connection??]  or  some   similar

message.  You simply need to exit your com program.  Exiting

kermit without logging off  will simply close the connection

and typically log you off.

You may  select a specific terminal  emulation which TCPPORT

should  try  to  run  by  setting  the  tcpterm  environment

variable before running tcpterm:

eg.  set tcpterm=vt102

Advanced WATTCP.CFG Options          Advanced WATTCP.CFG Options

This  section  is  useful  once  you  have  determined  that

Waterloo TCP actually works for you.

Including Sub-Config Files

You may wish to use a combination of generic WATTCP.CFG

file  and  a  smaller  sub-config file  which  will  be

located  on  the  user's  private  subdirectory.    Any

command which can be placed in the main config file may

also be placed (or replaced) in the sub-command file.

eg.

include = c:\local.cfg

After the subcommand file  is parsed, Wattcp returns to

the  main config  file.   The depth  of this  system is

limited by  the number  of file  handles and  the stack

size.

If  the  subcommand  file  cannot be  found,  an  error

message will  be printed.   To allow for  the possible,

but not-essential existance of a file (i.e., include it

if  it is there, but don't  complain otherwise) you may

simply prepend the filename with a question mark.

eg.

include = ?c:\local.cfg

IP Addresses

Most network  administrators would  prefer to  not have

many  copies of  the configuration  file, but  rather a

single  file  from   which  everyone   can  be   easily

configured.

As  demonstrated above,  Waterloo TCP  normally accepts

the ip number from within the WATTCP.CFG file.

BOOTP

Many  sites prefer  to use  BOOTP, a  standard protocol

which  requests  the  user's   ip  address  and   other

information from a BOOTP server.

To use BOOTP, you must specify the name 'bootp':

my_ip = bootp

in the config file.  This will broadcast the request on

the local subnet.   You  may specify  a specific  BOOTP

server which need not be on the same subnet, by using:

bootp = host

eg.  bootp = 129.97.128.1

The  default  timeout value  is  30 seconds.    You may

change that by using:

bootpto = seconds

eg.  bootpto = 50

If no  WATTCP.CFG file is found,  Waterloo TCP programs

always resort to BOOTP.

ETHERNET to IP Table

Another option currently  exists, I  allow multiple  IP

numbers in WATTCP.CFG  with each  one being  tied to  a

particular Ethernet address.  If your  Ethernet address

is found in list, your IP address will be assigned.

ETHIP=ethaddr,ipaddr

eg.  ETHIP=00:01:2F:BC:44:33,128.252.35.4

In  this   case,  the  machine  with  Ethernet  address

00:01:2F:BC:44:33  would  be  assigned  the  ip address

128.252.35.4.    Note   that  Ethernet  addresses   are

hexadecimal with intermediate  colons, ip addresses are

dotted  decimal, and I use a comma to separate the two.

Also, since  Waterloo TCP removes white  space, you may

place  a space between any  of the fields  if you don't

use quotes, and  you may  end the line  with a  comment

describing  where  the  station  lives or  to  whom  it

belongs.

You can quickly find the Ethernet  address of a station

by running the TCPINFO command.

Subnets

The Internet is comprised of many, many subnets.  There

are several  protocols normally used to  help computers

reach computers on other subnets.

Most PC based  TCP kernals depend on  routing tables to

manage the possible  routes, so I  elected to use  that

strategy.

A  routing  table  exists  in  memory  with  a  current

capacity  for 12  different  routes.   Each route  must

specify a gateway, an  optional destination subnet, and

then an optional subnet mask.

gateway = gate_ip [, subnet [, subnet_mask ]]

eg.  gateway = 129.97.176.1        # default

eg.  gateway = 129.97.176.2, 129.97.0.0, 255.255,0,0

The  first  example  shows  how a  default  gateway  is

created.  A default gateway is used if no other choices

exist.

The second example shows how to specify a gateway for a

particular subnet.  In this example, whenever the 'top'

16 bits are 129.97, that gateway will be used.

Yes,  you need not always  specify the mask,  but it is

necessary for class B subnets, so I simply suggest that

you always do specify the mask.

You  may specify  the same  gateway several  times with

different routes.

Non-contiguous subnet bits are supported.

To check  your configuration and to  see the precedence

of gateways, run TCPINFO.EXE.

Host Name

Some applications will wish  to know your PC's name,  a

short  textual  name.    This   may  be  set  with  the

WATTCP.CFG line:

hostname = name

eg.  hostname = mole

Notice that  you do  not  specify the  domain, that  is

found from the domain string.

Timeouts

Most Waterloo  TCP programs  have  a specified  timeout

value between activity  before a timeout error  occurs.

For  example,  the maximum  response  time  to an  open

request  before the  connection is  given up  should be

reasonably  long so  that distant  connections will  be

usable, but short enough that the user will not believe

the computer has hung.

Applications may specify  their own timeout value,  but

if  they  chose  to  use  the  system  default (all  my

applications do), the default value may be set from the

WATTCP.CFG file.

SOCKDELAY=seconds

eg.  SOCKDELAY=40

The  default value is 30  seconds.  A  smaller value is

unwise,   but  larger  values   may  be  necessary  for

particularly bad connections.

Maximum Segment Size

If you understand MSS and know what you would like, you

can change it:

MSS=bytes

eg.  MSS=512

Cookie Server

You may specify a cookie server in the  WATTCP.CFG file

with the line:

cookie = server

eg.  cookie = 129.97.128.1

eg.  cookie = sunee.uwaterloo.ca

Up to 10 separate cookie servers may be added.  TCPINFO

will  list  them  all.    BOOTP will  also  add  cookie

servers.

BOOTP Features and Limitations

BOOTP is not the greatest  method of configuration.  In

fact  there  is   currently  a  committee   looking  at

implementing its successor.

Waterloo  TCP  programs  will  automatically  get  many

configuration parameters from the BOOTP server if those

values are returned:

IP address

subnetmask

gateway        (only one will be added)

nameservers    (all supplied will be added)

cookieservers  (all supplied will be added)

hostname

The domain name cannot be specified currently.   Of the

gateways,  only one is recorded by Waterloo TCP as they

do not indicate subnets or anything else useful.

Notes:          Notes:

The most up-to-date versions  of these files, their sources,

and  new programs  are  available  on Sunee.uwaterloo.ca  by

anonymous FTP.  Check out pub/wattcp.

All  executables   there  are  copyrighted  but  are  freely

available for use and non-commercial distribution.

The library files which do the actual tcp communications are

also there.   They too are copyrighted,  but may be used  in

commercial and non-commercial work.   You are free to  do as

you choose.   If you intend to program  with this package, I

would  highly  recommend  the  developers  manual  described

below.

Developers  may wish to join  the Waterloo TCP mailing list,

join by mailing to:

wattcp@Development.Watstar.UWaterloo.ca

The  programmers manual includes  examples, a full reference

of the approximately 50 functions, notes on conversions from

UNIX.  The  cost is $40 ($US if you live in USA, $Cdn if you

live in  Canada.  $40 US  for anywhere else.   Make check or

money order payable to :

Erick Engelke

1010-130 Lincoln Rd.

Waterloo, Ont., Canada

N2J-4N3

The  proceeds are  entirely used  to offset  the cost  of my

manuals  and  software  costs  necessary for  improving  the

package.    The next  step is  Windows  DLL's, but  I cannot

afford everything I need to do that.

I have mentioned the  public domain CUTCP and NCSA  programs

which  do  an  excellent  job  of  TELNET,  RSH  with  VT100

emulation,  and  much more.   You  may  wish to  compare the

programs and use the ones which work best for you.

For their executables, use anonymous ftp to:

omnigate.clarkson.edu    128.153.4.2     for CUTCP

and  ????                     128.174.20.50   for NCSA

I hope that this distribution helps you in some way, and I'd

like to thank the contributors,

Bruce  Campbell who  wrote  the original  program  from

which tcpport was  derived.  He also wrote the DOS

network I log onto every morning.

Tim  Krauskopf's  NCSA Name  Domain  code  was used  to

develop Waterloo TCP's resolve function.

Edmund J. Sutcliffe donated a good portion of BOOTP.

Jim Martin made a  lot of extensions to  Edmund's BOOTP

work and was influential in the new nameserver and

new gateway code as well as the COOKIE stuff.

Jason Dent found some bugs and helped optimize WATTCP's

performance.

Dean Roth  found  some  low  level  bugs   and  greatly

improved the FTP program.

Although countless others have  given me good ideas and

noticed an incorrect line  here or there, but none

have been  more  thorough or  helpful than  Tarjei

Jensen.

If you would like  to add your name to  the programmers

list,  send me a copy  of your program  and I'll gladly

include it in the distribution with full credit.

Erick Engelke

Erick@development.watstar.uwaterloo.ca

Waterloo TCP Architect

July 8, 1992