T.A.P. Version 2.0 (C) 1989-1995

(Triode Amplifier Program)

FREEWARE

Courtesy of KD9JQ

When using the TUBEDATA.EXE File Utility you will be queried

for the device parameters in the following order.

The 3CX5000A7 is used as an Example.

Name      =  3CX5000 (Name of File, use DOS conventions

.TUB extension automatically added)

VB Max    =  6500    (Maximum Plate Voltage Rating)

VB Low    =   400    (From Grounded Grid Constant Current

Characteristics curves.

VB High   =  1100    (From Grounded Grid Constant Current

Characteristics curves.  ~ 2 to 3 * VB Low)

Gain      =   200    (Tube Amplification Factor)

Cin       =  56.5    (Grid-Cath  Capacitance in pF)

Cout      =  34.0    (Grid-Plate Capacitance in pF)

PDiss     =  5000    (Max Plate Dissipation of Device)

GDiss     =    50    (Max Grid  Dissipation of Device)

Cooling   =  Forced Air  (Enter description of required Cooling Method)

"The following requires the user to use interpolation of the curves

to select the Grid Voltage & Grid current per IB.  Determine which

IB (Peak Plate Current) curves are to be used.  8 IB's are required,

as are the corresponding EG, IG inputs per VB Low, VB High.

Enter Grid Voltages as a positive value."

Input IB:  2,3,4,5,6,7,8,9

Input EG @ VB Low:  50,67.5,92.5,110,127,145,166,178

Input EG @ VB High: 42,60,82.5,95,110,125,141,155

Input IG @ VB Low:  .088,.163,.355,.474,.744,.956,1.218,1.75

Input IG @ VB High: 0,.005,.01,.02,.065,.113,.167,.232

"Once the last input is done a file can be generated in the

correct format for the TRIODE.EXE program."