JAMBO - Memory Booster Option, Version 1.0

JAMBO is a Device Driver which is designed to make more memory available

for you using DOS 5.0 with its EMM386.EXE memory manager.

JAMBO is a provided as a fully working Shareware product by

Adlersparre & Associates Inc.

manufacturers of the Dynamic Memory Control package which can load and

remove device drivers and TSR programs from conventional and Upper memory

memory without rebooting. If you register JAMBO (registration fee is

$10.00), you will receive a coupon to purchase Dynamic Memory Control at

50% off (a $40.00 savings), as well as the latest version of JAMBO when it

is available.

JAMBO gives you several different options to get more memory in the Upper

Memory Blocks (UMB) area (the area past the 640k boundary). You can

increase the Upper Memory available by as much as 92k, and have this 92k

adjacent to your current UMB, making it 92k bigger.

Quick Memory Summary

--------------------

Most systems have free memory in the area past 640k. A normal memory layout

would be something like this:

Conventional Memory:

+------------------+------------------+

|                                     |

Programs     |              0000-9FFF              |

|                                     |

+------------------+------------------+

Upper Memory:

+------------------+------------------+

EGA/VGA      |              A000-AFFF              |

+------------------+------------------|

Mono Buffer  |    B000-B7FF     |    B800-BFFF     |   Color Buffer

+------------------+------------------|

Video Code   |    C000-C7FF     |    C800-CFFF     |   Unused

+------------------+------------------+

Unused       |              D000-DFFF              |

+------------------+------------------+

Unused       |              E000-EFFF              |

+------------------+------------------+

BIOS         |              F000-FFFF              |

+------------------+------------------

Usually, under DOS 5, EMM386.EXE creates the Expanded Memory page frame at

D000-DFFF, but you can use the FRAME=E000 option to force the Expanded

Memory page frame at E000 instead.

Looking at the above diagram, if you could free up the area called BIOS,

you could add an extra 64K of upper memory, and if you could move the video

code page at C000-C7FF somewhere else, you could get another 32k, making a

total of 96k.

JAMBO can let you do this, by using the HMA, which DOS normally uses. There

is a tradeoff, of course. You will use about 40k more of conventional

memory, since you cannot use DOS=HIGH in CONFIG.SYS (only with the /BH or

/VH options of JAMBO). But, you get up to 92K more of upper memory by

sacrificing 40k of conventional. And your upper memory is one big area, for

loading network drivers, CD-ROM, and any other programs. You can get as

much as 220k of upper memory.

The way JAMBO works is simple. You include JAMBO.EXE as a driver in your

CONFIG.SYS file at boot-up time after DEVICE=HIMEM.SYS and before

DEVICE=EMM386.EXE, and specify either one or two parameters. The parameters

can cause JAMBO to swap the video code page at C000 (hex) to somewhere

else, and the BIOS code page at F000 (hex) to somewhere else. The

"somewhere else" is defined by one of the following:

DEVICE=JAMBO.EXE /VH   swaps the video code page to the HMA. This

option only works if you are not using DOS=HIGH

to load DOS buffers into the HMA.

Using the /VH option puts the video code page

in faster memory. This option cannot be used

with the /BH option.

DEVICE=JAMBO.EXE /VA   swaps the video code page into Expanded Memory.

If you use this option, optimization is done

for the video code page, to make the video

display faster. No conventional memory is used

if you select this option.

DEVICE=JAMBO.EXE /VN   swaps the video code page into Expanded Memory,

but no optimization is done. This makes for

slower screen displays, but may be better for

some programs which use graphics. Use this

choice only if /VA does not work for you.

DEVICE=JAMBO.EXE /VM   swaps the video code page into the monochrome

buffer at B000 (hex). This option can only be

used if you do not use any software which tries

to use the monchrome buffer. Most systems which

have a color display can take advantage of this

option.

With all of the above options, you can then include the video code page at

C000 (hex) when you use EMM386.EXE, to make it into Upper Memory, available

for loading TSR's and device drivers. You use the "I=" parameter to do

this, such as "I=C000-C7FF".

DEVICE=JAMBO.EXE /BH   swaps the BIOS code page at F000 into the HMA,

giving you another 60k to use with EMM386.EXE.

The "/BH" option lets you load the BIOS code from F000 into the HMA (High

Memory Area). This will let you allocate the area from F000-FEFF (not

F000-FFFF) for use by TSR's and device drivers. When specifying the

parameter to EMM386.EXE, use:

DEVICE=EMM386.EXE RAM I=F000-FEFF

The last 4k must be left alone (that is from FF00 to FFFF).

Remember that the JAMBO.EXE device driver MUST always be in CONFIG.SYS

BEFORE "DEVICE=EMM386.EXE", and AFTER "DEVICE=HIMEM.SYS". The following

examples show you how to set up JAMBO in different configurations.

Example 1 - swapping the video page to Expanded Memory

------------------------------------------------------

Suppose you are using EMM386.EXE (the memory manager which comes with DOS

5.0). To swap the video page to Expanded memory, and give yourself an extra

32k, use the following commands in CONFIG.SYS:

DEVICE=HIMEM.SYS

DEVICE=JAMBO.EXE /VA

DEVICE=EMM386.SYS RAM FRAME=E000 I=C000-C7FF

DOS=HIGH,UMB

Note the I=C000-C7FF option on the EMM386.SYS line. This tells the memory

manager to include what was the video code page at C000, and make it usable

Upper Memory. JAMBO has moved the video code page into Expanded Memory.

The 32k extra will be added to the front of the UMB area which the memory

manager used to pick up for you.

We specified FRAME=E000 here because the memory manager usually likes to

place the Expanded Memory frame at D000, and E000 is more advantageous,

since you can get more memory in a single UMB that way.

Example 2 - swapping the video page to the High Memory Area

-----------------------------------------------------------

The High Memory Area (HMA) is where DOS locates much of itself if you use

the DOS=HIGH option in CONFIG.SYS. That usually saves 40k to 50k of

conventional memory. But if you would rather have 32k more in Upper Memory,

for loading networks, or other big TSR's or device drivers which won't

otherwise fit, JAMBO gives you the option of relocating the video code page

to the HMA. You would do it like this:

DEVICE=HIMEM.SYS

DEVICE=JAMBO.EXE /VH

DEVICE=EMM386.EXE RAM FRAME=E000 I=C000-C7FF

DOS=UMB

In this case, we don't use DOS=HIGH,UMB, just DOS=UMB, so that DOS will not

load itself into the HMA. But we do tell the memory manager to include the

video page which we have swapped to the HMA (C000) with JAMBO.

Doing this causes DOS to use more conventional memory, but gives you a

bigger Upper Memory region.

Example 3 - swapping the video page into the Monochrome Buffer

--------------------------------------------------------------

Many systems have a color monitor, so there is a 32k unused area at B000

(hex) which JAMBO can use for the video page. If you have a color monitor,

and you don't run programs that use the monochrome area, you can pick it up

as an area for the video code page.

Why is that an advantage? Because swapping the video code page with the

monochrome buffer gets you a bigger (32k bigger!) UMB instead of having a

separate 32k area, and another area. That means you can load TSR's and

device drivers in one big UMB area instead of having to juggle programs to

see which will fit best in each UMB.

To use the monochrome buffer, you would use:

DEVICE=HIMEM.SYS

DEVICE=JAMBO.SYS /VM

DEVICE=EMM386.EXE RAM FRAME=E000 I=C000-C7FF I=B000-B7FF

DOS=HIGH,UMB

In this example, we include the video code page with the I=C000-C7FF

option, and also include the monochrome buffer with I=B000-B7FF, for JAMBO

to use.

Example 4 - swapping BIOS into the HMA

--------------------------------------

You can swap the BIOS in page F000 into the HMA, giving you an extra 60k of

upper memory (not 64k, because the last 4k at FF00-FFFF must be left

alone). With this option, you can not use DOS=HIGH in CONFIG.SYS, but you

do get a much bigger high memory area. You would use the following

parameters:

DEVICE=HIMEM.SYS

DEVICE=JAMBO.EXE /BH

DEVICE=EMM386.EXE RAM I=F000-FEFF FRAME=E000

DOS=UMB

This would put the Expanded Memory page frame at E000, splitting up the

region. A better way to do this would be:

DEVICE=HIMEM.SYS

DEVICE=JAMBO.EXE /BH

DEVICE=EMM386.EXE RAM I=D800-FEFF FRAME=C800

DOS=UMB

Where the Expanded Memory page frame is at C800 instead, and you get a

bigger single contiguous region.

Example 5 - going for as much as possible

-----------------------------------------

If you were to load the video code page into Expanded Memory, and load the

BIOS page into the HMA, and include the monochrome buffer as upper memory,

you would have a 220k of available Upper Memory in two regions (the color

buffer is between them at B800-B8FF). You could use the following:

DEVICE=HIMEM.SYS

DEVICE=JAMBO.EXE /BH /VA

DEVICE=EMM386.EXE RAM I=C000-FEFF I=B000-B7FF FRAME=C000

DOS=UMB

Notes about JAMBO

-----------------

JAMBO has been tried on many computers successfully, including

Packard-Bell, DELL, Toshiba, IBM and several other clones, but in our

experience, we can truly say that "no two clones are alike". If you

experience a problem with JAMBO, we would like to hear from you so that we

can make it a better product.

JAMBO is provided as a Shareware utility. Please pass it on to others to

try, however no commercial fee may be charged for its use. JAMBO is a

copyright software product of Adlersparre & Associates Inc.

You are encouraged to register this software product, to take advantage of

the special offer to purchase Dynamic Memory Control at half price, and to

receive the latest version of JAMBO, when it is available.

Dynamic Memory Control can load and unload TSR's and device drivers from

conventional and upper memory, and convert device drivers and TSR's into

self-removing programs. It also has several memory maps, system displays, a

full-screen editor, pull-down mouse supported windows, and will even create

batch files to automate your applications. And you can get DMC for only

$39.95 (a $40.00 savings) when you register this copy of JAMBO.

Call for info on other products from Adlersparre & Associates Inc.

Adlersparre & Associates Inc.

501-1803 Douglas Street

Victoria, BC, Canada, V8T 5C3

Phone (604) 384-1118  or  FAX (604) 384-3363