Tandy DAC Programming Guide

version 1.7

September 11, 1994

This guide is by:

Jeffrey L. Hayes

130 Forest Ave., Lot 1

Orono, Me. 04473

(207) 866-7903

Internet:  tvdog@delphi.com

The information in this guide is based on the _Tandy 1000 TL Technical

Reference Manual_, Ralf Brown's interrupt list, the Tlslsnd package by

Bruce M. Terry, Jr., John Ball's "Tandy Sound Programming Information," and

miscellaneous sources.  If you have additional information, or you see an

error that should be corrected, please call or write.  Particularly, if you

have technical information about a Tandy model with the DAC, and what you

have does not agree with this, let me know.  Please distribute this guide,

and please include Tandy DAC support in your programs!

All programs and documentation included with this package are hereby placed

in the public domain.

Contents

--------

1.  Introduction

2.  The Machines

3.  BIOS Interrupts

4.  I/O Ports

5.  Setting the Recording Rate

6.  Playing in Direct Mode

7.  Program Examples

8.  Other Sources

1.  Introduction

On July 27, 1988, the Tandy Corporation announced two new computers, the

8086-based 1000SL and the 80286-based 1000TL.  These systems contained a

number of innovations remarkable for the time, one of which was the

capability of recording and playing digitized sound.  Tandy's news release

declared:

Tandy's innovative reputation continues with the 1000 TL, as the

company also introduces high fidelity audio "input and output"

technology to the PC industry.

Through the use use of internal digital to analog and analog to digital

converter circuitry, the 1000 TL is the first MS-DOS personal computer

to record, store and playback speech or sound.

With this innovation, Tandy has given software publishers one of the

most exciting new tools yet with which to create quality products not

available to the industry today.

... and all that most PC-compatibles could do straight from the factory was

"beep."  The same audio DAC circuitry was used in several subsequent Tandy

models, until Tandy started including Soundblaster capability instead.

Unfortunately, software publishers other than Tandy itself do not appear to

have made much use of these "exciting new tools" - at least, publishers of

free- and shareware have not.  Users have been sold on the sound

capabilities, only to discover later that DeskMate Music and Sound are

(almost) the only applications available that support their sound chip.

There is a Windows 3.1 sound driver for the Tandy chip, available from

Radio Shack, but many machines with the Tandy DAC can't run Windows 3.1.

There were also several games written to support the chip, some of which

will not run on newer models due to bugs in the chip detection routines.

Music and Sound are fine programs, but they suffer from the usual Tandy

curse: they are proprietary.  Sound .snd files are playable on a Tandy and

no other machine (Conv2snd v2.00 will convert between .snd and .wav, and

Snd2sam, in this package, will convert .snd files to Amiga .mod samples).

Music likewise creates song modules that no other machine can use, and in

this case there is nothing to convert a standard song format such as .mod

to .sng, or vice versa.

Furthermore, both programs run only under DeskMate.  On the 1000SL and TL,

the core of DeskMate is in ROM and is tied to DOS 3.3, also in ROM.  Hence,

upgrading DOS results in losing DeskMate and consequently Music and Sound.

One can obtain the standalone version of DeskMate and use Music and Sound

with that, but only at additional expense.

Confronted with this lack of software support for the built-in sound, users

desiring sound capability better than that supplied by the PC speaker

(programs that use the generic PC speaker for sound output will, of course,

run) have had to purchase Soundblaster cards.  Personally, it offends me

that I have to use one of my computer's scarce expansion slots to get good

sound, when it already has perfectly adequate (though no longer state-of-

the-art) sound capability built in.

The offense is all the more offensive when one considers that the original

Soundblaster card is a downgrade from the built-in sound.  Both the Tandy

DAC and the Soundblaster play 8-bit mono, but the Tandy can play at 44kHz

and record at up to 22kHz (higher rates are possible, but it becomes

difficult to set the rate precisely at those speeds), while the Soundblas-

ter can only play at 22kHz and record at 13kHz.  The Tandy sound chip also

has built-in BIOS support on those machines that have it, making it easy to

program.  (Tandy's use Phoenix BIOSes.)

2.  The Machines

The following Tandy models include the Tandy DAC: 1000SL, SL/2, TL, TL/2,

TL/3, RL, RL-HD, RLX, RLX-HD, RLX-B, RLX-HD-B, and RSX, and the entire

2500-series.  (Thanks to Ken Udut for this information.)  At the time some

of these models were produced (1988-89), Tandy was the number-one

manufacturer of PC's in the U.S.  The 1000RSX and the 2500SX were current

as of 1993.  Tandy's are very reliable machines, and hundreds of thousands

of machines with the Tandy DAC are still in service.

I don't have a lot of information about the 2500's; most of what follows

refers mainly to the 1000-series.  The 2500XL to RSX are 16-bit AT-class

machines.  The 2500SX and submodels are based on the 80386SX and use the HT

chipset.  There are 25MHz and 33MHz 80386's in the 2500 line.  2500's can

have more than 5MB of memory.

The processors used in the 1000's vary from the 8MHz 8086 in the 1000SL to

the 25MHz 80386 in the 1000RSX.  Except for the 1000SL and SL/2, machines

that have the Tandy DAC have at least an 80286.  The SL, TL, and RL-series

are XT-compatibles.  The RSX is an AT-compatible.  The RLX is a borderline

case, being an AT on the motherboard, but having an (that's right, *an*) 8-

bit expansion slot.

The 1000RLX's have built-in VGA with 256k RAM (not expandable).  The

1000RSX has SVGA (AcuMos AVGA2 with 256k upgradable to 512k).  All models

can be upgraded to VGA.  The SL's, TL's, and RL's have proprietary video

which is a superset of CGA.  Specifically, they have Tandy Video II, which

is a superset of Tandy Video I (used in the earlier 1000-series), which is

compatible with PC-Jr video, which is a superset of CGA.  The video on

these machines is comparable to what one would get from an EGA card with a

CGA monitor attached (640x200 16-colors).  Tandy Video II also supports

Hercules graphics when a mono monitor is attached.  (I have programming

information about Tandy Video II if anyone is interested.)  The 2500SX has

SVGA and also supports Hercules; I would assume that other 2500 models also

have at least VGA.

All of the 1000-series with the Tandy DAC have a 101-key keyboard, though

on some the keyboard is of the unidirectional XT type (don't count on being

able to send commands to the keyboard).  Earlier models without the DAC

used an 84-key keyboard with nonstandard scan codes, but the scan codes are

standard on the newer models.

In addition to the DAC, there is a 3-voice tone and noise generator

compatible with the Texas Instruments SN76496 used on the PC-Jr.  Tandy

offered some enhancements over the PC-Jr 3-voice chip, including the

capability of using the DAC as a 4th tone channel and the ability to play

tones an octave lower than the TI chip.  The presence of this 3-voice tone

capability has caused some confusion.  The Tandy DAC does not have 3

voices.  While the 3-voice circuitry is on the same IC as the DAC, they are

separate functions.  Older 1000's without the DAC still have the 3-voice

circuitry, but without the 4th channel or the lower octave.

Some (all?) of the computers under consideration have a volume control

knob.

The DAC uses DMA channel 1, IRQ 7.  IRQ 7 is shared with the printer; the

BIOS takes account of this fact.

All models came with DOS 3.3 or later.  There is a rumor that it is

impossible to upgrade the DOS version on some 1000 models; this rumor is

false, and by now most have been upgraded.  The 1000RSX and all models of

the 2500 can run Windows 3.1; earlier models can run Windows up to 3.0

(upgrades being required in most cases for adequate performance).  OS/2

will run only on the 1000RSX and the 2500-series.

3.  BIOS Interrupts

The BIOS provides access to the DAC on Tandy's that have the chip, using

Int 1Ah.  This is the easiest way to program the chip.  The BIOS announces

completion of a sound request by calling out to Int 15h.  See the comments

following the list.

--------------------------------------------------------------

Int 1Ah AH=7Fh                  Get Sound Chip ID

(Tandy 2500 only)

Use:

AL = location for chip ID info:

0 = on stack

1 = in buffer at ES:BX

if AL=1 then

ES:BX -> buffer for chip ID info

Returns:

Sound chip ID string (???).

Int 1Ah AH=80h                  Set sound multiplexer for audio source

(PC-Jr / Tandy 1000 only)

Use:

AL = source:

0 = 8253 channel 2

1 = cassette interface

2 = audio in on I/O board

3 = sound chip

Returns nothing.

(This function is a no-op on the 1000TL.)

Int 1Ah AH=81h                  Get DAC base I/O port

Use:

AL = 0

Returns:

AX = base I/O port of DAC (00C4h on most 1000's, 01E4h on 2500's).

CF clear if sound not busy, set if busy.

(AX is unchanged if the function is not supported.  Use this call to

detect the DAC.  The 3-voice chip is at (DACbase - 4) if this call is

supported, 0C0h otherwise.)

Int 1Ah AH=82h                  Record sound

Use:

ES:BX -> buffer for sound

CX = length of buffer

DX = transfer rate (1-4095)

Returns:

AH = 0

CF clear if sound not busy, set if busy.

Int 1Ah AH=83h                  Play sound

Use:

ES:BX -> buffer for sound

CX = length of buffer

DX = transfer rate (1-4095)

AL = volume (0-7, 0 = silence)

Returns:

AH = 0

CF clear if sound not busy, set if busy.

Int 1Ah AH=84h                  Stop sound

Use nothing.

Returns nothing.

--------------------------------------------------------------

I don't know the format for the ID string returned by Int 1Ah AH=7Fh on the

2500-series.

Int 1Ah AX=8003h should be called before using the 3-voice tone and noise

generator on the Tandy 1000 or PC-Jr.  It is not necessary to call this

interrupt to use the DAC.  Also set bits 5 and 6 high and bit 0 low at port

61h on older machines without the DAC before using the 3-voice chip.

Use Int 1Ah AH=81h to detect the presence of a Tandy DAC.  To do this,

first issue Int 1Ah AX=8003h.  This is "Set Multiplexer for Audio Source"

on the 1000SL/TL and IBM PC-Jr.  As shown above, the audio source is being

set for the 3-voice tone generator.  On the TL, this function is a no-op;

all audio (from whatever source) is mixed.  At any rate, it does no harm.

The real purpose is to detect the presence of PCMCIA Socket Services, which

is present if the interrupt returns CX=5353h.  If PCMCIA services is

installed, the DAC can't be used because of a software interrupt conflict,

so the detection routine should report that the DAC is not present in this

case.  Then issue Int 1Ah AH=8100h.  If AX is changed, a Tandy DAC is

present (unsupported BIOS calls leave registers unchanged).

The value returned in AX by Int 1Ah AH=81h is the base I/O port of the DAC.

Generally, it is not necessary to program I/O ports to use the DAC;

however, it _is_ necessary to do so if you wish to use the DAC in direct

(non-DMA) mode (see below).  The base port may be either 0C4h or 1E4h, as

noted above.

If Int 1Ah AH=81h is supported, the address of the 3-voice tone and noise

generator is computed by subtracting 4 from the base DAC port address

returned; i.e., the 3-voice chip is at either 0C0h or 1E0h.  If the call is

not supported, the 3-voice chip may still be there.  All models of the

Tandy 1000 have the 3-voice chip, even if they don't have the DAC.  On

older Tandy 1000 models with the 3-voice circuitry, the byte at FFFF:000Eh

is 0FFh and the byte at FC00:0h is 21h.  The 3-voice tone and noise

generator is at port 0C0h on the older machines.

The BIOS calls out to Int 15h with AX=91FBh when sound DMA is complete

(i.e., when Int 1Ah functions 82h, 83h, and 84h are complete).  Your

program must hook this interrupt.

Sound buffers do not have to be on 64k boundaries.  The BIOS masks the DMA-

across-64k problem from the program.

The value returned in the carry flag by the various sound functions is not

to be relied on.  In general, if CF is set, it means that the sound is

busy, but if CF is clear, it does *not* mean that the sound is available.

Initializing the sound chip seems to be the main problem to be overcome

when using the Tandy DAC.  One must call AH=84h and wait for the BIOS

callout to be sure; see Playwav.asm, included with this package.  (Thanks

to Bruce M. Terry, Jr., for his information on this.)  Int 1Ah AH=84h is

listed above as returning nothing, but in fact it "returns" a BIOS callout

to Int 15h when complete.

Note that the above method of initializing the DAC will hang systems

without the chip.  It is important to detect the chip before initializing

it.

The DAC should be finalized before the program exits by the same method as

it is initialized.  If this is not done, the Tandy BIOS will not set the

DMA channel 1 count to terminal count, which can cause problems with other

devices using DMA channel 1.

The transfer rate values in DX used by Int 1Ah functions 82h and 83h are

dividers used to clock the sound chip.  Unfortunately, the values used are

different in each function, and the values used with function 82h depend on

the model involved.  For function 83h, compute the divider as follows:

divider := 3579545 / sampling_rate_in_Hz

The DAC is clocked at 3 times the PIT rate.  Parenthetically, the divider

for function 83h is exactly the same as the period value in an Amiga music

module.

Tandy says that for function 82h, you should compute the divider as

follows:

divider_for_82h := divider_for_83h / 10     (on a TL)

divider_for_82h := divider_for_83h / 11.5   (on an SL)

Unfortunately, the above seems to cause sounds to play back slightly faster

(about one semitone higher in pitch) than they record.  The true

relationship on the TL is found experimentally to be approximately:

divider_for_82h := (divider_for_83h - 12.5) / 10

For the SL, I compute the recording divider as follows (for no reason other

than symmetry with the above):

divider_for_82h := (divider_for_83h - 12.5) / 11.5

I think (wild guess) that other models are likely to follow the TL for the

divider when recording, but it would probably be wise to make this an

option.  Note the 1000SL and SL/2 are the only 8086-based models with the

Tandy DAC.  See the section below for more on dividers and sampling rates.

Int 1Ah AH=84h may be used at any time to stop sound input or output.

The 1000SL can be detected as follows; I don't know about detecting the

SL/2.  First check for a Tandy 1000-series BIOS signature:  if the byte at

FFFF:000Eh is 0FFh and the byte at FC00:0h is 21h, the system is a Tandy

1000-series.  If so, issue Int 15h, AH=0C0h to get the system ID.  If the

interrupt returns with carry set, the call is not supported, and the system

is not an SL (or any other model with a Tandy DAC, most likely).  If carry

is clear, ES:BX addresses machine ID information.  On the SL, the word at

ES:[BX+2] is 00FFh.  See Recwav.pas for example code.

For seamless playing and recording of long sounds, Int 1Ah can be called

from within the Int 15h handler.

4.  I/O Ports

As noted above, it is not generally necessary to program I/O ports to use

the Tandy DAC.  One case where it is is if you want to perform output byte-

by-byte rather than using DMA (see "Playing in Direct Mode," below).  The

3-voice chip also lacks BIOS support and must be programmed directly.  The

sound circuitry (3-voice tone and noise generator and DAC) use a range of 8

I/O ports, either 0C0h-0C7h or 1E0h-1E7h.  See the above section on "BIOS

Interrupts" for how to determine the port addresses.

--------------------------------------------------------------

Port 0C0h (1E0h)                SN76496 3-voice tone and noise generator

Write only

Bits

7 6 5 4 3 2 1 0       Function

---------------       --------

1 x x 0 y y y y       Set tone frequency, channel xx (0-2).  yyyy = low 4

bits of divider.

0 w z z z z z z       (following the above) zzzzzz = high 6 bits of divider.

If extra divide enabled (port 0C4h bit 6), w = high

(11th) bit of divider (otherwise, w is a "don't

care" bit).

Use the following:  OUT 0C0h,AX  where

AX = 0wzz_zzzz_1xx0_yyyy

1 x x 1 y y y y       Set tone attenuation, channel xx (0-2).  yyyy =

attenuation (1's complement of volume).  Attenuation

1111b = no sound.

1 1 1 0 x y z z       Set noise type/frequency.  x is a "don't care" bit.

y = periodic/white noise code:  0 = periodic noise,

1 = white noise.  zz = frequency:  0 = N/512, 1 =

N/1024, 2 = N/2048, 3 = Tone 3, where N = 3579545.

0 is highest noise frequency (see below).

1 1 1 1 y y y y       Set noise attenuation.  Attenuation 1111b = no sound.

Port 0C4h (1E4h)                DAC Mode Register

Bits 0-2 and 4-6 are read/write

Bit(s)        Meaning

------        -------

0-1         DAC function select:  0 = joystick, 1 = sound channel (with

3-voice chip), 2 = successive approximation (sound input),

3 = direct write to DAC (sound output).

2           DMA enable (if bit 1 = 1).  0 = DMA disabled, 1 = DMA enabled.

3 (read)    1 = DMA interrupt has occurred.

3 (write)   DMA interrupt clear.  0 = DMA interrupt held clear, 1 = DMA

interrupt allowed.  When DMA interrupt occurs, this bit

must be set low, then set high to allow another interrupt.

4           DMA interrupt enable.  0 = DMA EOP interrupt disabled, 1 =

DMA EOP interrupt enabled.

5           Sound divider sync enable.  0 = synchronization disabled, 1 =

synchronization enabled.  See below.

6           Sound chip extra divide enable.  0 = extra divide disabled

(10-bit dividers for 3-voice tone generator), 1 = extra

divide enabled (11-bit dividers).

7 (read)    0 = Successive approximation done.  When polling for input,

indicates that a byte of sound is ready to read at port

0C5h (1E5h).

7 (write)   reserved

Port 0C5h (1E5h)                Sound Control Register (if 0C4h bits 0-1 = 1)

Read/write

Bit(s)        Meaning

------        -------

0-2         DAC sound channel duty cycle (pulse width):

0 = 6.25% duty cycle

1 = 12.5% duty cycle

2 = 18.75% duty cycle

3 = 25.0% duty cycle

4 = 31.25% duty cycle

5 = 37.5% duty cycle

6 = 43.75% duty cycle

7 = 50.0% duty cycle

3-5         reserved

6-7         DAC waveshape select

0 = pulse

1 = ramp

2 = triangle

3 = reserved

Port 0C5h (1E5h)                Direct Read/Write Register (if 0C4h bit 1 = 1)

Read/write

If using polling with sound input, read sound data from this port.  If

doing sound output, read current output byte from here, or write a byte

to here for output to the DAC.

Port 0C6h (1E6h)                DAC Frequency LSB (if 0C4h bits 0-1 <> 0)

Read/write

Least significant 8 bits of divider for frequency of DAC when used as a

sound channel.  Least significant 8 bits of divider for frequency of

sound DMA when doing sound input or output.  See below.

Port 0C7h (1E7h)                DAC Amplitude/Frequency MSB (if 0C4h bits

Read/write                        0-1 <> 0)

Bit(s)        Meaning

------        -------

0-3         Most significant 4 bits of divider for frequency of DAC when

used as a sound channel.  Most significant 4 bits of divider

for frequency of sound DMA when doing sound input or output.

See below.

4           reserved

5-7         Sound amplitude (if port 0C4h bit 0 = 1).  7 levels, appx.

3 dB per level.  0 = no sound.

--------------------------------------------------------------

On older Tandy 1000's, bits 5 and 6 at port 61h must be set to enable the

sound chip (bit 6 enables the keyboard and must always be high).  Bit 0 at

this port should be low (this disables timer channel 2 output to the

speaker).  Do not change the other bits; on the 1000TL, setting bit 4

disables the speaker.  It is not necessary to program port 61h on newer

systems with the DAC.

There are various ways of disabling the sound chip and/or the speaker,

depending on the system.  On the 1000TL, the speaker is disabled by setting

bit 4 at port 61h, and the sound chip is disabled by clearing bit 2 at port

0FFEBh.  On the 2500SX, the sound chip can be disabled by setting bit 6 at

port 73h, then clearing bit 4 at port 1F9h - or by clearing bit 2 at port

1FBh.  The speaker and sound chip will normally be enabled, however.

The 3-voice tone and noise generator is addressed at any of a set of 4 I/O

ports.  In other words, if the chip is at 0C0h, the same data can be

written to 0C1h, 0C2h, or 0C3h instead, to the same effect.

The tone frequency for the 3-voice chip is 111,860Hz divided by the 10- or

11-bit divider.  To get 11 bits of division, one must set bit 6 at port

0C4h (1E4h); this bit is clear by default.  11-bit dividers are only

available on systems with the DAC installed.

With periodic noise code 0, frequency 0 gives a tone at 466Hz (A# above

middle C), frequency 1 gives a tone at 233Hz (A# below middle C), frequency

2 gives a tone at 116.5Hz (second A# below middle C), and frequency 3 gives

a series of clicks.

With white noise code 1, frequencies 0, 1 and 2 give white noises at

successively lower frequencies, and frequency 3 gives an intermittent noise

something like radio static.

Bit 5 at port 0C4h (1E4h), if set, will cause all 3-voice tone frequency

registers to be reloaded when port 0C6h (1E6h) or port 0C7h (1E7h) is

written to.  This makes it possible to guarantee that the tone frequencies

are in phase.  This bit is low by default.

The base frequency for the 12-bit divider programmed in ports 0C6h (1E6h)

and 0C7h (1E7h) varies according to the DAC function selected by bits 0-1

at port 0C4h (1E4h).  When using the DAC as a sound channel, a base

frequency of 111,860Hz is divided to obtain the output tone frequency.

When using the DAC for sound output with DMA, the base frequency is

3,579,545Hz.  For sound input with DMA, well, it depends (see section on

"Setting the Recording Rate" below).

The volume programmed in port 0C7h is used for sound output, either

digitized sound or tone channel.  The volume is ignored for sound input.

Note that providing a stairstep wave for the joystick comparators is a

function of the DAC.  You can't play digitized sound at the same time you

read the joystick (although you could use the 3-voice tone and noise

generator).  A future release of this file will contain information about

the joystick ... maybe. :-)

5.  Setting the Recording Rate

The above note on setting the recording divider is only an approximation.

There seems to be no stable relationship between the recording divider and

the playback divider, or between the recording divider and the sampling

rate.  The situation for recording is thus different that that for playing

- the formula for the playback divider given above is quite reliable.  See

Divider.doc for experimental data and more accurate means of setting the

divider.  Recwav times the DAC at various dividers to set the recording

rate, which works fairly well now (see Tspak.doc) - the timing subroutine

has been replaced in this version.  Another possibility is to use the data

in Divider.doc to construct a lookup table (i.e., playback divider vs.

recording divider, using the playback divider formula).

This package includes a program, Testdiv, to record and play back short

snippets of sound using various recording and playback dividers.  You can

use Testdiv to hear what different divider values sound like.  The program

is largely self-explanatory.  The sound buffer can be saved to disk as file

Testsnd.ub, a headerless unsigned PCM sound file (Raw2wav can convert it to

.wav if you want).  Note that if you select "Increment playback divider and

play" or "Decrement playback divider and play" from the menu, the playback

divider will be changed before playing, but the new value won't be

displayed until after playback is done.

6.  Playing in Direct Mode

It is possible to play sounds on the Tandy DAC in direct mode, that is,

byte by byte, if that is what your program requires.  Direct writes to the

DAC are not supported by the BIOS, so it is necessary to program the DAC

registers directly to enter this mode.  Once the chip is programmed for

it, however, the Tandy DAC can be treated as just a dumb homemade DAC made

out of a bunch of resistors and attached to a parallel port.

There are obvious advantages and obvious disadvantages to direct mode.  DMA

mode access to the DAC through the BIOS is cleaner, faster, and easier to

program than byte-by-byte access, and it supports higher sampling rates and

better sound quality.  On the other hand, if you are considering adding

Tandy DAC support to an existing application that already supports homemade

or Covox DAC's, direct mode may be the easiest way for you to do so.

This package includes a program, Setdac.asm, which sets the Tandy DAC for

direct mode playback.  Setdac doesn't do anything by itself; it is designed

to be run in a batch file before another program that uses a homemade or

Covox DAC.  Use of Setdac is described in detail in Tspak.doc.  Once Setdac

is run, any program can write 8-bit unsigned PCM data to the DAC's I/O port

byte-by-byte to generate sounds.  At noted in Tspak.doc, Setdac only

provides for direct mode output of sound.  It is possible to program the

Tandy DAC for direct mode input if anyone can come up with a reason for

doing it, but one can't switch from input to output or vice versa without

reprogramming the chip.  Refer to Setdac.asm when reading the following.

The DAC is programmed for direct mode in several steps.  First, detect the

Tandy DAC as described above.  The value returned in AX for Int 1Ah,

AX=8100h is the base I/O port of the DAC.  Verify that the value returned

addresses a set of read/write I/O ports by writing to the port at (base+2)

and reading back from the port.  You should be able to set and clear all

bits.  Next, initialize the DAC by polling Int 1Ah functions 84h and 81h,

as for DMA mode.

Next, program the DAC itself.  The DAC Mode Register is its base I/O port.

Its bit mapping (for writes) is as follows.  Bits 0-2 and 4-6 are

read/write.  To write to the DAC byte-by-byte, bits 0 and 1 of the base

port must be set, bits 2-4 must be clear, and bits 5-7 should be left

alone.

Finally, set the output volume.  The ports at (base+2) and (base+3) are

used to do this.  These ports are read/write.  When using DMA with the

Tandy DAC, port (base+2) and bits 0-3 of port (base+3) are used as a

divider to determine the DMA transfer rate (see above).  I don't know

whether they matter with direct writes or not; I thought it best to set

them to 0.  Bit 4 of port (base+3) is reserved; I set it to zero as well.

Bits 5-7 of port (base+3) determine the output volume; this is the value

programmed in AL for Int 1Ah function 83h.  I set the volume to the

maximum; as noted above, most Tandy's have a volume control knob - and most

programs provide volume control.

If all went well, the DAC is now set to receive 8-bit unsigned samples at

port (base+1), just as if it were another dumb DAC.

7.  Program Examples

Playwav.exe is my RIFF WAVE player for the Tandy DAC.  It is an example of

a working application that uses the chip for output with DMA.  Playwav is

getting a little long in the tooth now (it was the first program I wrote

for the DAC), but it does work.

Recwav.exe is a RIFF WAVE recorder for the Tandy DAC.  It is an example of

an application that uses the chip for DMA-mode input.  This version of

Recwav is not a very good example, though, in that it bypasses the BIOS and

programs the sound hardware directly.  (Recwav does that to be able to stop

the input at any time and see how much has been recorded - information the

BIOS does not supply.)  Testdiv still uses the BIOS interface, though, and

you can use Playwav as a guide for buffer switching while recording through

the BIOS.

Setdac.com sets up the DAC to perform direct-mode (byte-by-byte) output.

It doesn't perform any output itself (there's a small click when it's

setting up the chip); it is meant to be run in a batch file before another

program that uses a DAC on a parallel port.

Piano.com is a little time-waster demonstrating the use of the Tandy 1000 /

PC-Jr 3-voice tone generator (a Tandy DAC is not required).  This version

has been changed to use Int 9 rather than Int 15h AH=4Fh, so it should

actually be runnable on an old 1000 (unlike the previous versions).

Programs in this package are written in either Turbo Pascal v.5.0 or A86

assembler (or both).  A86 is a shareware assembler by Eric Isaacson,

available on Simtel.  Assembler programs and subroutines in this package

will require modification to be assembled by Masm or Tasm.  .exe programs

written in assembler are linked with the Microsoft Object Linker.  There is

a short .mak file included for each executable file; the .mak file is a

text file describing how to compile or assemble and link the program.

8.  Other Sources

Bruce M. Terry, Jr's., Tlslsnd.asm is a set of subroutines providing access

to the DAC chip.  It was a major source of information for this guide.  The

routines are designed to be called from Microsoft QuickC.  The package is

freeware.

John Ball's Playsnd.exe is a program to play .voc, .wav, .iff, and Tandy

.snd files on either the Tandy DAC or the PC speaker.  His documentation

and letters were an important source, especially regarding the 2500-series.

John has recently decided to release the source code for Playsnd into the

public domain.  He also has a program, Sbplay.exe, to play the same types

of files on the Soundblaster or PC speaker; source code for Sbplay is not

available, however.

Joseph Albrecht's Tandysnd.com is a TSR providing access to the 3-voice

tone and noise generator through the BIOS video interrupt (Int 10h).

Interface modules for Turbo Pascal, Microsoft QuickBasic, and Spectra

PowerBasic are provided.  This program is freeware.

John W. Ratcliffe's IBM Digitized Sound Package contains a TSR,

Tansltl.exe, which can be called to play sounds in the background on the

Tandy DAC.  There is also a program, Play.exe, to play sounds in a batch

file.  This program is commercial ($19.95); demo is free.  $250 fee for

commercial use.

(All of the above are available from me by email upon request.)