REMOTE

A  program  to  transfer ASCII files between a TANDY Model 100 and

and an IBM PC, also allows remote DOS operation.

This program will run an an IBM or compatible that  has  an  rs232

port.  It  will operate on port 1 or port 2,and will use any baud rate

from 300 to 9600. It is intended for use with the Model 100 from Radio

Shack  for  ASCII  only  file  transfer  but  may be useful with other

similar units. The only protocol allowed is XON/XOFF. The cable should

be wired as follows:

IBM   Model 100

2  -  3

3  -  2

7  -  7

If  you  wish  to  use  the remote DOS commands, you must add some

jumpers to the IBM side. Connect pin 4 to 5, and connect pins 6  to  8

to 20.

To  start  up  you may just enter the name REMOTE or you can enter

one or both options. The options allow you to set the baud rate and/or

select  the port ( these can be changed later ). The defaults are 2400

baud on port 1.

Enter options as:

REMOTE [BAUD] [#]

For baud use 300, 600, 1200, 2400, 4800, or 9600.

For port use 1 or 2.

The port can't be entered without a baud.

The IBM screen ( local ) will show the time, the revision  number,

and  the  current  baud  rate  and  port  number. The baud rate may be

changed by pressing 'B'. The rate will increment each  time  until  it

reaches  9600,  it  will  then  start  over  again  at  300  baud. The

communications port number is toggled between 1 and 2 by pressing 'C'.

Put the M100 in 'TELCOM' and set the stat to  6,8,n,2,e  for  2400

baud.

You can EXIT and return to DOS at any time by pressing 'ESC'.

The Model 100 ( remote ) will show a menu of commands:

| ? Help  |C Chgdir|D Dwnlod|F Files |

| J JmpDOS|L Log   |P Print |U Upload|

Free space = ( disk drive free space )

Drive/Pathname>

The '?' command gives a brief description of the commands.

The  'C'  command  will  allow  you to change your directory path,

similar to the DOS 'CD' command.

The 'D' command will download an ASCII file to the M100  unit.  It

will ask for the name of the file to send and open the file to send to

the M100. You must then press 'DOWN' on the M100, and enter  the  name

of the M100 file to write into. When ready, hit a key on the M100, the

file will now be sent to the M100. When the data stops, YOU must press

'DOWN' again on the M100 to stop the file capture. Then press a key at

the M100 to exit the download function. NOTE: During the download, the

screen will lock to prevent scrolling, this improves the speed.

The  'F' command lists the files in the current directory. It will

ask for a wildcard to use in the search, or you may just press  return

for  all files. The directory is displayed 3 across with a pause every

6 lines. Just press a key to continue.

The  'J'  command  requires  that  the  'DOS.BAT'  file  and  your

'COMMAND.COM'   be   present  in  the  current  directory  or  in  the

environment PATH. You will be asked if this is what you  want  to  do,

press  'Y'.   You will now be setting at a dos prompt and at this time

you can perform many of the common DOS commands. You should  be  aware

that many programs bypass DOS and will not function properly so should

be avoided. To return to the REMOTE program type 'EXIT'.

NOTE: The program and the batch file  'DOS.BAT'  should  reside  in  a

directory  that has a path to it, this is only required if you want to

use the JUMP TO DOS feature. If you don't need this feature,  you  may

place the program anywhere convenient and you won't need the 'DOS.BAT'

file.

The 'L' command will LOG you onto another  disk  drive.  You  must

select  a  valid drive, the program will try to verify your selection.

If you select an invalid drive you may 'HANG UP' the system.

The 'P' command is used to print an ascii file from the  M100.  If

the  printer  isn't  ready,  the function will tell you so.  Press the

'UP' function on the M100 and enter the  file  name,  for  WIDTH  just

press  return.  The data will be sent, buffered, and printed. When the

file is sent, you must wait for the  buffer  to  empty  out  into  the

printer.

The 'U' command is for UPLOADING ascii files to the disk. You must

enter a name for the disk file, if the file exists you will  be  asked

if  you  want  to  delete  it. Press the 'UP' function on the M100 and

enter the M100 file name to send, for WIDTH  just  press  return.  The

data  will  be  sent,  when done you must wait for the disk file to be

closed out.

NOTE: Most functions can be aborted by pressing the space  bar  on

the IBM.

Although I haven't tried it, you should be able to use a modem and

call up your IBM as a simple form of remote  dos.  If  you  try  it  I

recommend you set your modem to no codes and to answer on first ring.

This  program  is  public domain and may be given freely to anyone

that may have a need for it. It was written in 'C'  and  linked  to  a

library  with  some simple async functions. The async is NOT interrupt

driven, so the IBM has trouble keeping up. This means that  you  can't

run a background task ( as PRINT or an on-screen clock ) and upload at

the same time. I hope to add an interrupt driven module in the future.

Also  remember  that  this program only transfers simple ascii without

error checks.

If you have any needs that aren't covered be this version,  please

contact  me. Anything that will improve the program will be considered

for future releases ( I can use  them  myself  ).  Leave  me  a  message

(  good or bad ) on Compuserve my PPN# is 75775,40 or on the SEOCS-BBS

( 614-439-7451 ) leave a message to Wayne Orwig.

###########################################################################################################