*** Release Notes for HIT-IDE.SYS Ver.1.06 ***

1. General

HIT-IDE.SYS is the device driver for DOS CD-ROM Extensions that supports

ATAPI CD-ROM drives.

2. Command line switches of 'HIT-IDE.SYS'

(1) SYNTAX

DEVICE = [x:][\dir\]HIT-IDE.SYS /D:(device name) [/N:(number of drives)]

[/T:(drive type)] [/S:(base I/O address)] [/V]

(2) Meaning of switches

/D:(device name)         Specifies "Device Name".

/N:(number of drives)    Specifies number of drives to be supported

by this driver.  Values 1 to 8 are valid.

If this switch is omitted, the driver will

search for all connected drives.

NOTE:  To speed the booting process, use

/N:1 if you only have one drive connected.

The search will stop after the first drive

is found.

/T:(drive type)          Specifies the first drive type to begin to

support.

Definition of drive types are as follows:

/T:   base I/O address    master/slave

---------------------------------------------

0     1F0h(primary port)  master

1     1F0h(primary port)  slave

2     170h(2nd port)      master

3     170h(2nd port)      slave

4     1E8h(3rd port)      master

5     1E8h(3rd port)      slave

6     168h(4th port)      master

7     168h(4th port)      slave

If this switch is omitted, it is the same as

/T:0.

NOTE:  To speed the booting process, use

/T:1 if you know your drive is connected

to the primary slave.  Use /T:2 if your

drive is connected to the secondary master.

The search will start at this number and

continue until /N:(number of drives) are

found.

/S:(base I/O address)    Change IRQ number to the specified base I/O

address.

base I/O           IRQ No.

address    without /S:    with /S:

(default)    (alternate)

----------------------------------------------

170h            15              10

1E8h            12              11

168h            10              9

(1F0h)          (14)            (14)

/V                      Display installation information.

NOTE:  Use the /V option to show you where

your drive(s) are connected.  Then you can

use the /T: and /N: options to speed your

booting process in the future.

3. Examples of installation

<Case 1> -- Default is automatic search for all drives connected.

DEVICE = HIT-IDE.SYS /D:MSCD001

-- Install all drives connected to primary, 2nd, 3rd & 4th.

<Case 2> -- Using /N: switch to limit the maximum number of drives.

DEVICE = HIT-IDE.SYS /D:MSCD001 /N:2

-- Install maximum of 2 drives while auto searching,

beginning the search with Primary Master.

<Case 3> -- Using /T: switch to change the start of automatic search.

DEVICE = HIT-IDE.SYS /D:MSCD001 /T:2

-- Search beginning at /T:2 (2nd port and master).  Continue

searching through /T:7.

<Case 4> -- Using /N: and /T: switches together.

DEVICE = HIT-IDE.SYS /D:MSCD001 /T:2 /N:2

-- Search beginning at /T:2 installing maximum 2 drives.

<Case 5> -- A single drive is connected to Primary Slave.

DEVICE = HIT-IDE.SYS /D:MSCD001 /T:1 /N:1

-- Search beginning at /T:1 installing only 1 drive.

<Case 6> -- A single drive is connected to Secondary Master.

DEVICE = HIT-IDE.SYS /D:MSCD001 /T:2 /N:1

-- Search beginning at /T:2 installing only 1 drive.

<Case 7> -- Using /S: switch to use alternate IRQ numbers.

DEVICE = HIT-IDE.SYS /D:MSCD001 /S:170,168

-- Auto search all drives using alternate IRQ numbers

for 170 (IRQ 10) and for 168 (IRQ 9).   Use default

IRQ numbers for others (1F0 IRQ 14; 1E8 IRQ 12).

<Case 8> -- Using all switches together.

DEVICE = HIT-IDE.SYS /D:MSCD001 /T:2 /N:1 /S:170

-- Install only 1 drive starting search at 2nd port master.

Use alternate IRQ of 10 for 170.

4. For Windows 95

If you are using the CDR-7730 drive on a DOS/Windows 3.x system and

you upgrade your system with Windows 95, Windows 95 will automatically

upgrade your system appropriately and the CDR-7730 will work well.

If you are using Windows 95 and then adding the CDR-7730 drive, please

run the "CDSETUP.EXE" program from the installation disk for the CDR-7730

under Windows 95 and restart your system.  After this, the CDR-7730

will work well.

The above specifications are subject to change.

November 10, 1995

Hitachi America, Ltd.