E2000plus-series Ethernet Adapters

Installation file for

\NDIS.30\WINNT31 directory

============================================================================

********************************************

*  Driver Installation for Windows NT 3.1  *

********************************************

1.  Logon as Administrator and select the Network icon from the

Control Panel.

2.  Select "Add Adapter..." button and highlight the NE2000 compatible

driver to be installed. If Windows NT does not offer an NE2000 driver,

choose the "<Other> Requires disk from manufacturer" item.

3.  Insert the Windows NT system or E2000plus driver diskette into a

floppy disk drive.

4.  Follow the instructions appearing on the screen and select the

appropriate drive name when required to give the location for the

driver and the OEMSETUP.INF file.

For example :

A:\NDIS.30\WINNT31 <-- for E2000plus Driver

5.  After pressing Continue, confirm the driver diskette, choose

IRQ and/or IO Address, and click on OK.

6.  Now, the driver has been installed. Select OK button from

the Network window. The protocols and driver will bind together.

7.  Reboot