RING V3.2

(c)1999 Stefan Peichl, Heidelberg, Germany

http://home.t-online.de/home/stefan.peichl

eMail: stefan.peichl@t-online.de

RING uses the palmtops hidden feature of detecting GSM signals.

If such signals are on the air, RING simply plays a ring sound.

If you start RING from SysMgr or a SysMgr DOS shell, make sure

that you selected 'serial port powered' in DOS from the Setup

program of the palmtop, otherwise RING will not run.

You don't have to worry about serial power if you call RING

from plain DOS.

Put your mobile or cordless phone close to the palmtop. If you

receive a call, or if you make a call, the palmtop plays a nice

ring tone and shows "recording call" on the screen, which is

indeed not true.

RING also works, if your palmtop is close to the phone of

another person, which could be fun, but which could also end

up in serious trouble if the person looks at your screen.

RING should work as long as the phone is not farther away from

your palmtop than 50 cm. However this distance may vary from

phone to phone and also depends on the distance between phone

and next receiver/transmitter station.

To exit the program, press any key. However if you press one of

the keys 0-9, these values are returned as errorlevels and may

be used in a batch file to start special actions at the time

you 'pick up' your virtual phone. You may for example play a

WAV file saying "hello" if you 'pick up' with key 1.

DEBUG MODE

If you call RING with any command line switch or press Enter

from the opening screnn, RING will enter DEBUG mode.

In DEBUG mode, the serial port is set up for infrared and both

power supplies for infrared are switched on.

The 3 volt power supply from GPIO pin 0 (Hornet index register

51 hex, bit 0) is responsible for sending out IR data.

The 5 volt power supply from GPIO pin 15 (Hornet index register

52 hex, bit 7) is responsible for receiving data from the photo

diode.

The setup is: 9600 baud, 8 data bits, no parity, 1 stop bit.

In DEBUG mode, any detected EMI (electomagnetic interference)

causes a click to make it audible. In addition, an error

counter is incremented and the numer of occurred errors of

the last second is displayed in the upper half of the screen,

if there were any. The display stops, if no errors are detected.

On the first line of the screen you will see the mean value

per second of these errors. This value is important in order

to find out, if a shielding of the palmtop has any effect.

Additionally, on the first line you will also find the absolute

number of framing and overrun errors of this run. These are

only of statistical significance.

In the lower half of the screen are the incoming characters

displayed, as received by the UART.

Any key you press is send out to the IR port and immediately

received by the photo diode and hence echoed on the lower half

of the screen. Use this feature to test, if a manipulation of

the IR hardware had a negative effect on the IR receiver, which

would result in no echo or a falsified echo.

Indeed every echo also increments the error counter, because

the software cannot now, if the character came in through the

IR port or if it was induced by EMI.

If you want to know, if a hardware manipulation of the palmtop

had success in suppressing EMI but still allowing IR communi-

cation, you should see no data coming in if a mobile phone is

online, but any key you press on the palmtop should be echoed.

During a test run, you may always press the space bar to reset

all counters and start a new measuring.

Pressing the Enter key brings you back to the 'ring' mode.

Press ESC to quit the program.

NOTE: Light sleep is disabled in DEBUG mode. Always quit the

program or leave the mode, otherwise your batteries burn

down quickly and may cause loss of your RAM content.

DISCLAIMER

This software is copyrighted freeware.

The author assumes no liability for physical damages, direct or

consequential, which may result from the use of this software

to yourself or to anybody else.