SUBJECT: 7" Power Supply  5volt Power Upgrade

Device references are from the schematic located in the Tech Reference.

Rigged up a test jig to monitor voltage

and current under various static load conditions. I had intended to take

supply to failure, but I chickened out at 3.65 amps @ 4.83v (17.6watts)

when the heatsink after 6 hours burned my finger. I concluded that the

7" card +5v supply was rated at a safe 3 amps but failure would surely

occur at 4 amps.

I then hooked up the 11" card to the same load. No sweat! Increased load

to 4.65 amps @ 4.94v (22.9watts). Maintained same load for 6 hours.

Judged heat sink heating to be safe and normal. I concluded that the

11" card +5v supply was rated at a safe 4.5 amps with failure probably

occurring at approximately 5.5 amps.

Next step was to attempt to achieve a 1.5 amp improvement in the 7" card

(+5 volt supply only. The +12 volt supply would not be altered.)

After further analysis of the circuit I concluded that resistor R5 was

the current sensor. The darlington power device handles the bulk of the

regulation power dissipation. About 50 ma off the top is handled by the

Z2 precision regulator chip.

The Q2 power darlington 2N6040 was replaced by a higher power

device with very similar characteristics. An ECG-271 was mounted on

top of the heatsink (drill one hole) with two additional heatsinks

(Two Radio Shack Part number 276-1363's work nicely.)

mounted top and bottom as would fit in the case without touching same.

Three reasonable gauge multi-strand wires were connected in place of

the original darlington transistor in the original holes.

Heat transfer compound and an insulating spacer was used.

With no modification to the R5 current sense resistor the supply was

limited to 3.5v. Halving the value of R5 with a parallel .05 ohm 1watt

resistor allowed 4.83v @ 4.6amps. After 6 hours the heat sink was

quite warm but well within reason.

(If a .05 ohm resistor cannot be located, a .1 ohm 1watt will work.)

The four 3 amp rectifiers were also quite warm however, making it

apparent that they might fail when the +12v supply has normal load and

why the 11" card uses a heavily heat-sinked bridge package for this

purpose. I replaced these with four 6 amp devices.

The four bridge 3a rectifiers can be replaced with Motorola

MR750 (6A). Radio Shack 276-1661 is a set of four (6a @ 50v).

The remaining limitation is the 56 va brick vs the 75 va.

The 75 va brick has a 7 amp in line fuse as well as a thermal fuse

built into the transformer.

The 56 va brick has a 5 amp in line fuse and a thermal fuse. This

lower fuse rating is the factor which limits this modification to

only one additional sidecar.

Several users have overcome this limitation by paralleling the primary

and the secondary of two 56va transformers such that the power available

is double while keeping the output voltage the same. When doing this the

phasing is critical. You most make sure that you hard wire the primary

and secondary leads to prevent accidentally supplying twice the normal

voltage. Dangerous voltages are present on the primary of the brick.

So exercise extreme caution if you choose to parallel two bricks.

The power expansion sidecar which is similar in design to the 11"

internal power supply card uses the 56 va brick.

I believe the transformer will hold but don't put a blanket

over either transformer or the thermal fuse will blow.

Just for info, CR5 SCR is a crowbar device. If for any reason the +5 or

+12V supplies go too high, (CATASTROPHIC FAILURE OF Q2 OR Q1 IN SHORT MODE)

it causes a high current through R19 causing F1 to fail.

Also for info, the 30v 1 watt zener CR6 is a "catch diode" which serves

the purpose of passing inductive spikes to protect Q1 and Q2.

Most common failure of 7" supply is the combination of a shorted

Q2 with a sometimes shorted CR6 and a blown F1 5amp pico-fuse.

When encountering these conditions, look for a short or overload (too many

sidecars) on the +5v supply.

Also the -12V supply is only used by com port applications. It is really

-6v on the 7" card and -12v on the 11" card.

All the documentation that was printed says only one sidecar is allowed

without the external power expansion. However no updates were made after

the 11" card was released. ThE 11" card clearly puts out about 1.5 amps

more on the +5v power line. That supports about any two additional

sidecars over the one specified for a total of three sidecars.

As far as IBM Boca is concerned there is no

difference between these two JR cards. The supply of new 11" cards is

depleted. Orders will be filled with whichever rebuilt or new card is

in the bin. The IBM solution for more than one sidecar is the power

sidecar.

The above modifications are simply an attempt to extend the life of

7" cards in a beyond design (beyond recommendations) situation. If you

make any modifications to the card, it will be ineligible for exchange.

Making the above mods will allow you to safely add at least one sidecar

for a total of two sidecars. Adding any more is purely at your own

risk.

If failures of F1 fuse only occur, it is probably because of using

the internal (infernal) modem as well as the speech sidecar both of

which along with the diskette drive draw on the +12v supply.

This is the only case where one sidecar exceeds the 7" card (+12v)

capacity. The only solution is to remove the internal modem or the

speech sidecar or add a power sidecar.

If you have to replace the +12v Regulator chip Q1 (a 1.5amp @ 15watt

device). Replace with an identical package 3amp @ 30 watt device.

RCA part number SK9337 or Sylvania ECG1912. Use heat sink compound.

This will eliminate having to replace the device again should an

excessive load be present. The overload will simply blow the fuse.

Question? Can the -6v supply be upgraded to -12v on the 7" card?

Answer: It is simply not practical.

The -6v supply is dependant on sufficient steady current from the +5v

supply. It is developed by transformer step-up of the +5v supply ripple

current. You would have to increase the number of turns in the L1 secondary.

It would be an increased drain on the already taxed +5v supply even with

the upgrade modification. A more practical solution for your application

would be a $7.00 or so Radio Shack plug in wall D.C. powersupply. The

current required is less than 250 ma.. Notch the upper rear flange of JR's

case to allow passage of the power cable. Remove CR1 from the 7" card.

Connect the negative lead to the anode lead hole of CR1 and the positive

lead to a convenient ground point.

Further caveats: Any modifications are done at your oun risk. Any

warranties if any still are in effect will be voided.

No responsibility is assumed by any person or

corporation for any damage caused by these

modifications.

SUBJECT: PCJR POWER SIDECAR NOTES

In reply to several requests for more service info on the PCJR power

sidecar...the following:  Schematic references such as CR2 and L3

are generally found marked on the boards but may be covered by a

component. References such as A6 and B26 refer to the I/O connector.

On/Off control voltage (+5v) comes from System on line B26.

Thru protection diode CR2 to gate of Q2 an N-channel F.E.T..

Drain of F.E.T. is +5v when JR is off and at ground potential when

JR is on. Pin 6 of both L296 Regulator chips must be at ground for

ON operation of Regulators.

Input voltage to L296 regulator chips is on pin 3 (both at roughly +21v)

Output is on pin 2 thru inductors L2 or L3 (+5v & +12v respectively), to

A6,A26 or A2 respectively.

The -12v supply is derived off the +12v supply by 1:1 inductive coupling

in L3. The secondary voltage generated by ripple current on the D.C. line is

then rectified, filtered and provided to A30.

The L296 regulators used are identical despite the fact that they were

maintained as separate part numbers by IBM. The device is an extremely

reliable 4 amp regulator. In the rare event that one goes bad, a replacement

device can be obtained from Unitrode or SGS-Thomson. Make sure that you

get the correct package as two styles, flat mount and vertical mount are

available with preformed leads. The vertical mount will not work as the

inside leads are too short when reformed for flat mount as used in both

the PCJR 11" internal and the power sidecar.

Incidentally the Sylvania ECG replacement for this device will not work

because it is the vertical mount.

The power supply pins on the System side of the I/O connector (five of

them) have been removed as all voltages outboard are provided by the

power sidecar. Some users have restored these pins and hardwired the

drain of the F.E.T. to ground as a replacement for their defective

7" internal power card. The main problem with this is the loss of

the on/off switch. With the exception of the switch, this is equivalent

to the 11" internal card.

Hank Kennedy     Tie 678-6366        AUSVM7(HKENNEDY)