SUPER MORSE (TM)

ADVANCED TOPICS

Ver. 4.10

Copyright 1994

M. Lee Murrah

2-19-94

1. How Morse Code Is Defined..................................   1

a.DITS and DAHS..........................................   1

b. Morse Characters......................................   1

c. Code Weighting........................................   2

d. Code "Speed"..........................................   2

e. The "Farnsworth" Method...............................   3

2. Learning the Morse Code....................................   3

a. The Super Morse Teaching Method.......................   3

b. Studying Hints........................................   4

3. Scripted Disk Files........................................   5

a. The Message Command...................................   6

b. The Pause Command.....................................   6

c. Set Speeds Command....................................   6

d. Restore Speeds Command................................   6

e. Speed Lock Commands...................................   6

4. Customization of External Files............................   7

a. General...............................................   7

b. Data Files............................................   7

c. License Classes File..................................   8

d. Text Files............................................   8

5. QSO Templates..............................................   8

6. Equipment and System Requirements and Limitations..........  10

a. System Configuration..................................  10

b. Mouse.................................................  10

c. Code Speed Calibration................................  10

d. Video Display Card....................................  10

e. Disk Cache Programs...................................  11

f. Floppy Disk Write Protect Tabs........................  11

g. Windows Capability....................................  11

h. Speaker Volume........................................  11

7. Sound Boards...............................................  11

8. Interfacing External Equipment.............................  12

9. Circuit Diagrams for External Connections..................  13

a. Using an External Hand Key............................  13

b.  Connecting to a Receiver.............................  14

c. Driving a Transmitter or Other Device.................  14

d. Using a Volume-Controlled Headphone...................  16

e. Using an External Oscillator..........................  17

f. Using an External Sound Output........................  18

Page 1

This file contains information on selected advanced

features of Super Morse. See QUICK.DOC for basic information and

and overview of Super Morse.

1. How Morse Code Is Defined

----------------------------

a.DITS and DAHS

---------------

Morse code is a sound language that uses two different

lengths of a single tone in various combinations to represent

all the characters of the Morse alphabet. These two tones are

distinguished by their length, one a "short" tone and t he other

a "long" tone. The short tone is phonetically referred to as

"DIT", and the long tone as "DAH".  "DIT" is usually written as

"DI" when it is not the last element of a character. You will

often see the short tone represented as a "." (dot) and th e long

tone as a "-" (dash), but it is commonly agreed that use of these

symbols encourages a visual approach to the code, which slows

code learning.  You should never think of the Morse code as dots

and dashes or write it in that way!

The original Morse code as used in landline telegraphy was

represented by a combinations of timed clicks.  However, when

radio came into use, the code was revised to use tones which are

more suited for radio use. The original Morse code is used in la

nd telegraphy is called the American Morse code, but the code now

commonly used in radio is called the International Morse code.

b. Morse Characters

-------------------

The amateur radio Morse code set is a subset of the

international Morse code.  It uses a reduced character set of

the 26 letters, the 10 numerals, 4 punctuation marks and 5

special procedure signs.  The Morse character set support ed by

Super Morse is shown in the several figures that follow.

The printable Morse characters are shown in Fig. 1-1.  The

International Morse code that is used in amateur radio supports

a number of punctuation marks, but the amateur subset uses only

the five shown in Fig. 1-1.

Morse code communications uses several special characters

that convey procedural information in shorthand.  These are often

longer characters that can be viewed as combinations of

alphabetic characters without an intervening character space. The

mos t common of these procedure signs, or "prosigns", used in

amateur radio are shown in Fig. 1-2.  <KA> and <SN> along with

<BT> are frequently used in traffic handling.

Page 2

Several international characters are used in Morse code,

especially in Europe.  These characters shown in Fig. 1-3. are

also supported by Super Morse in some modes:

c. Code Weighting

-----------------

The various elements of Morse code are defined in relation

to each other. The DIT is assigned the shortest duration and is

defined as being one time unit in duration, and a DAH is 3 times

as long as a DIT, or 3 time units in duration .  The silence

between DITs and DAHs in a character, which we will call an

"element space" is the same length as a DIT, i.e., one time unit

in duration.  The silence between characters, which we will call

a "character space", is 3 time units in duration. Finally, the

space between words, which we will call a "word space" is 7 time

units in duration.

Perfectly sent code uses these standard relationships, but

code is not usually perfectly sent except when a computer is

used. Each person who sends code manually, develops a weighting

slightly different from the standard, and this unusual weighting

is said to constitute the operator's very recognizable "fist".

Operators who use automatic keyers can usually set the

weighting as a personal preference.  Weighting very far from the

standard sound choppy and hard to copy.

d. Code "Speed"

--------------

The speed of Morse code is measured in words per minute

(WPM). Code speed is an elusive measure since words are of

various lengths; and when random characters are being sent, there

are no words at all!

Code speed has therefore been arbitrarily defined by

standard words.  The standard word for plain text is "PARIS",

which, if you add up the time units in all the code sound

elements used to send "PARIS" (DITs, DAHs, and spaces between the

DIT's and D AHS, characters and words), you will find that it is

50 units of time long.  One word per minute is defined as sending

"PARIS" using code elements timed to fill one minute, including

the word space following the last word.  Ten words per minute

would be s ent at a speed necessary to send "PARIS" 10 times in

one minute.  The code elements are calibrated at any particular

speed against "PARIS", and when text is sent at, say, 10 words

per minute, more or fewer actual words may be sent depending

upon their len gth.

The word "PARIS" will not work for random characters since

the average Morse code character used to send English text is

shorter than the average character used to send random

characters.  This is because English does not use all the letters

Page 3

in equal numbers and because Morse code assigns the shortest

codes to the most frequently used letters in English.  The most

frequently used letter in English is "E", and it is assigned the

shortest code, the DIT.  A less frequently used letter such as

"Q" is ass igned a longer code, DAH DAH DI DAH.

For random characters, therefore, the standard word "CODEX"

was chosen. If you analyze "CODEX", you will find that it

contains 60 code elements. The effect of this difference is that

the DITS and DAHS in randomly sent characters should sound about

20 % shorter than those in plain text at the same nominal speed.

Randomly sent characters therefore should sound 20% faster that

plain words.  This is the reason that manufacturers of code

tapes can boast that the actual test will seem slow after using

thei r random character tapes!

e. The "Farnsworth" Method

--------------------------

Morse code is usually sent with the standard relationship

between character spaces and the elements which make up the

character.  In other words, the DITS, DAHS and element spaces are

sent according to the same speed standa rd.  However, Farnsworth

discovered that code learning can be accelerated if the DITs,

DAHs and element spaces are compressed as if the code were being

sent at a high speed while the character spaces are expanded to

maintain an overall lower send speed. This is called the

"Farnsworth method", and it is endorsed by the ARRL as the best

way to learn the code.

The Farnsworth method is supported and is used as the

default method by Super Morse.  Two Farnsworth speeds are

provided.  The "Lo" Farnsworth speed, preset at 18 WPM, is used

at send speeds below 20 WPM; and the "Hi" Farnsworth speed,

preset at 26 W PM, is used above 20 WPM. Beware, however, that

not all code tests are given in the Farnsworth format, or, if

they do, using these speeds ! Check with your testing

coordinator to determine which method will be used, and then use

that system when studying with Super Morse.

2. Learning the Morse Code

--------------------------

a. The Super Morse Teaching Method

----------------------------------

Unlike standard Morse code teaching methods, Super Morse

does not begin by introducing characters. Instead Super Morse

starts with sounds only -- characters are not taught or displayed

in the early lessons.  The idea is to emphasize that Morse code

is a sound language and to teach the fundamentals of code sound

s. Two sound-only exercises are provided. First, the basic DIT

and DAH sounds are taught. Second, Super Morse puts the basic

Page 4

sounds together into the four fundamental two-sound combinations.

Only after the basic sounds and sound combinations are

taught does Super Morse start identifying characters.  Before the

standard code groups are introduced, code "steps", which are

groups of two similarly sounding characters, are sent to

gradually i ntroduce the characters in pairs. The idea is to

teach the student to distinguish similar sounding characters from

the beginning, such that those which do not sound so much alike

will fall into place by themselves.  Also the characters are

slowly learned in an indirect fashion in that the student is

encouraged to concentrate on the differences rather than the

content of the characters themselves. Only when the steps are

completed are the standard code groups introduced.

The Learn selections "DIT/DAH" through "Levels" are ordered

to slowly build up your knowledge of the characters in a very

logical way.It is recommended that the Learn subcommands be used

in the same order as the selections are presented.

b. Studying Hints

-----------------

There is no one best way to learn the Morse code.  As with

foreign languages (to which Morse code is similar in some ways),

some people can learn the code faster than others.  The best

attribute that one can possess in learning the code is persistenc

e.

Super Morse implements one method of learning the Morse

code.  If you follow the lessons provided in the Learn phase, you

should have no trouble in learning the Morse code in a few weeks

or months.

In addition to the method provided by Super Morse several

general tips on learning the Morse code can be provided as

follows:

i.  Learn the characters by sound, not by sight.  Each of

the characters has its own characteristic length and rhythm.  Do

not memorize the characters using the visual dot and dash

symbols.  That will only slow you down and create bad habits that

wil l be very hard to break!

ii.  Set the character speed to 13 words per minute or

higher (the ARRL uses 18 WPM) and the send speed at whatever

lower speed you desire. (Super Morse automatically sets the

character speed to 18 WPM). Since your ultimate goal is 13 words

per minute , it is much better to learn the sounds of the

characters at the higher speed even though they are spaced at a

lower speed. As mentioned above this is called the Farnsworth

method, and it helps to eliminate the "hump" that most people

experience around 1 0 words per minute.

Page 5

iii.  Study every day, but if you feel you are stuck and

not making progress, take a few days off.  It's amazing how this

will sometimes clear your head.

iv.  Try to copy code in you head as much as possible.

v.  Obtain access to a ham receiver and try to copy real

transmissions off the air.  Some general coverage short wave

receivers permit reception of Morse code, but in general they

are harder to tune in on a Morse code signal.

vi.  As soon as possible learn the sounds of whole words.

You will find that whole words have characteristic lengths and

rhythms that distinguish them from all other words.

vii.  Turn on the Super Morse Build/Groups function and let

the computer send in the background while you are doing something

else, such as reading a book or working on a construction

project.  Try to identify characters while doing the other

activity. This will help your brain shift to "automatic" and will

help the code sounds become a natural "part of the wallpaper."

viii.  Although the code test no longer requires that you

demonstrate an ability to send Morse code, you should get a code

practice oscillator and practice your sending.  Note that Super

Morse can be used as a code practice oscillator using one of seve

ral keys on the keyboard or an external telegraph key.

ix.  As soon as you pass the novice test, get a rig on the

air and operate on the novice bands.  Nothing will increase your

speed and concentration as well as the pressure of an on-the-air

contact.  Don't worry about being perfect--the other guy is pr

obably in the same boat as you. II. Super Morse Basics

3. Scripted Disk Files

----------------------

The disk files that may be played with the Build|File

command amy be scripted by inclusion of commands that control the

operation of Super Morse. Scripted files permit Super Morse to be

automated for special learning methods, to prepare code tapes, or

to give tests.

Disk files are "scripted" by placing a period (".") in the

first column of any line followed by a command character in

capitals and a space and text, if applicable.  With scripting

Super Morse follows exactly the same line spacings as are used in

the script file. Thus, if you want lines skipped before and after

the message, place blank lines in your script file.

Scripting permits the following commands to be inserted in

a file.

Page 6

a. The Message Command

----------------------

The "M" command line as follows causes the <text> to be

printed in highlight on the screen:

.M This a message to the user

This will print "* This is a message to the user" on the screen.

b. The Pause Command

--------------------

The same effect as the message command, except with a pause

for you to press a key to continue, is provided by the "P"

command as follows:

.P Ask the user to do something

This will print "* Ask the user to do something" and instruct

you to press any key to continue.

c. Set Speeds Command

---------------------

Speeds can be programmed into the script at the indicated

send speed ("<ss>"), character speed ("<cs>") and word speed

("<ws>") with the "S" command as follows:

.S <ss> <cs> <ws>

d. Restore Speeds Command

-------------------------

The set speeds command automatically saves the current code

speed before it is changed.  The former speeds can be restored

with the following command:

.R

e. Speed Lock Commands

----------------------

The set speed keys can be disabled with the .L command and

re-enabled with the .U command.  These commands are not followed

by text.

Page 7

4. Customization of External Files

----------------------------------

a. General

----------

In addition to internal modifications, Super Morse permits

customization by changing the contents of Super Morse's text

files. Modifiable files all have the "SM" extension, such as

NAMES.SM.

If a word processor is used, the output must be ASCII text

only!  Most modern word processors output not only the text that

you enter in the document but also formatting information which

is not displayed on the screen. This invisible formatting info

rmation will cause to Super Morse produce garbage. If you use a

word processor, be sure to check your user manual to determine

how to save text in plain ASCII format without the formatting

information.

b. Data Files

-------------

Super Morse uses several files containing names, cities,

and other information used in QSO generation (simulated radio

contact), and other files for words used in word-based functions.

If you are not satisfied with the contents of these files, you m

ay change them to suit your tastes. Non-U.S. users will probably

want to change the contents of the states file to include the

provinces in their own country or other countries.

All data files require that only one item be placed on a

line starting in column 1 (not at the left margin of a word

processor file if the margin is shown!) and that no blank lines

be inserted in the middle of the file.

The names, antenna types, city names, and rig types can be

changed to reflect your desires by editing the files ANTENNAS.SM,

NAMES.SM, CITIES.SM, RIGS.SM, and JOBS.SM as described earlier in

this document. The maximum number and length of the entries are

as follows:

File           Max. Entries   Max. Length

ANTENNAS.SM           50             15

CITIES.SM           200              15

JOBS.SM             100              20

NAMES.SM            200              10

RIGS.SM             100              15

STATES.SM           100              15

Page 8

c. License Classes File

-----------------------

The only data file having a special format is the

CLASSES.SM file.  This file contains three lines in a special

order. The file distributed with Super Morse defines the United

States license classes and speeds as follows:

05 Novice

13 General

20 Advanced

The format is critical!  It must contain three lines and

start in column 1 (not the left margin of a word processor

file!). Each line begins with a speed in words per minute in

columns 1 and 2, followed by a space, followed in column 4 by the

name of the license class beginning at that speed, not exceeding

8 characters in length.  If a speed is less than 10 WPM, a "0"

must be placed at the beginning.  If only two license classes are

used in your country, repeat the second class definition on the

thir d line as in the following generic example:

07 Junior

11 Senior

11 Senior

If you have only one class of license, place the same

information on all three lines.  If your country uses more than

three classes of licenses, please let me know, and I will expand

this file.

d. Text Files

-------------

In some features Super Morse permits you to use a plain

ASCII text file as input.  An example in the Disk File command in

the Build phase.  These files should be created with a text

editor or word processor in the ASCII output mode as described

above .  The lines in these files can be any length and can start

in any column.

5. QSO Templates

----------------

The external template may be selected in the Generate exam

function. The external template is contained in a disk file

named QSOn.SM where "n" is a number from 1 to 99.  The file

QSO1.SM is provided as an example, but you may created other

ASCII fil es yourself.

An exam QSO template file comprises any desired text built

around up to 20 pieces of randomly selected data.  When data is

Page 9

desired in the QSO file, insert the identifying number of the

data type surrounded by curved brackets ("{" and "}").  The ident

ifying numbers are as follows:

1 - calling station      11 - countries worked

2 - responding station   12 - band 1

3 - rst report           13 - band 2

4 - name                 14 - job

5 - city                 15 - rig type

6 - state                16 - antenna type

7 - age                  17 - antenna height

8 - years a ham          18 - power (watts)

9 - license class        19 - weather

10 - states worked       20 - temperature (Deg. F)

The following is an example of a line in a QSO template:

My name is {4}, and I live in {5},{6}.

Super Morse randomly generates a name and substitutes it

for {4} and a city and state and substitutes them for {5} and

{6}.  An example of the result is as follows:

My name is Joe, and I live in Northfield, KS.

At least 10 pieces of data should be used in any exam file

to support the 10 questions that are asked in the exam.

Page 10

6. Equipment and System Requirements and Limitations

----------------------------------------------------

a. System Configuration

-----------------------

Your CONFIG.SYS file should authorize at least 20 files.

This can be done with the following statement:

FILES = 20

b. Mouse

--------

Super Morse supports a Microsoft mouse.  In some

configurations, Super Morse uses two serial ports. If you use a

serial mouse, there may be a conflict.  To fully use all Super

Morse features, three serial ports are required.

c. Code Speed Calibration

-------------------------

In order for Super Morse to send properly-timed code,

calibration to each computer is required.  Calibration is

affected primarily by the computer clock speed and memory

resident programs.

Super Morse uses two different calibration methods, (a)

clock and (b) loop, in order to handle various types of PC

designs and operating systems. The method that you desire to use

is selected from the Options menu.

Both methods require some calibration using the Options|Code

features. The timer method needs to be "tweaked" the first time

you use it to match itself to your computer, but thereafter it

self-calibrates to the computer clock speed each time the program

loads.  However, if you use the loop method, you must calibrate

Super Morse each time you add or remove a memory resident

program.

The loop method must be used if your computer does not

implement the system clock chip in the same way as the original

PC design, if you are running under Windows as a DOS application,

or if running on a PC emulator on a MacIntosh or other computer.

Super Morse is initially set for the clock method; but if the

code is erratic or choppy on your computer, you should change to

the loop method.

d. Video Display Card

---------------------

Super Morse supports CGA (color) and MDA (monochrome) video

displays only.  The proper display adapter can be selected from

Page 11

the Options menu. Super Morse will operate on EGA and VGA

monitors but in the CGA mode.

e. Disk Cache Programs

----------------------

Super Morse saves changes in the program parameters when

you exit the program.  If you are using a disk cache program,

the parameters may be saved to the cache area in memory and thus

may not permanently saved to disk.  If you a re having this

problem, you should try disabling any disk cacheing program which

you have installed on your system.

f. Floppy Disk Write Protect Tabs

---------------------------------

If Super Morse is run from a floppy diskette, a write

protect tab should not be used! Super Morse must be able to store

configuration and user information to the diskette.

g. Windows Capability

---------------------

Super Morse is written as a DOS program, but it will run

under Microsoft Windows under certain conditions.  The author

has confirmed that it will operate under the 386 Extended mode

(i.e., a 386 or 486 computer is required) using the loop timing

method.  You must set up a PIF file, and set the background to

foreground ratio to 10000 to 1 and set Execution to "Exclusive".

These are available as Advanced options in the PIF editor menu.

To save you time a PIF file to run Super Mor se under the

conditions stated is provided on the distribution diskette.

h. Speaker Volume

-----------------

The volume produced by the PC speaker is very low on some

PC's, but unfortunately there is no way to increase it without

an external amplifier. Some relief can be obtained by changing

the speaker tone since speakers (and your ears!) are frequency

dependent.  ADVICE: Get a Sound Blaster type sound card.  See

next section for setup.

7. Sound Boards

---------------

Super Morse now supports the Sound Blaster card and

compatibles. Super Morse initializes for use of the PC speaker,

and the Sound Blaster must be selected in the Options|Code menu.

A sound card is HIGHLY RECOMMENDED!

If you plan to use the Sound Blaster card for sound output,

you must use a text editor, such as EDIT or EDLIN, (or a word

processor in ASCII output mode) to insert a line in the

Page 12

AUTOEXEC.BAT file specifying an "environment string". The line

should b e in the following format:

SET BLASTER=A<addr> I<int no> D<DMA channel no> T<SB type>

For a Sound Blaster 1.0 using I/O address 220, Interrupt 7

and DMA channel 1, the string would look as follows:

SET BLASTER=A220 I7 D1 T1

The I/O address, Interrupt number, and DMA channel can be

determined using the TEST-SBC.EXE program provided with your

Sound Blaster.  If you did not change any of the jumpers on the

board when you installed it, you can use the factory default

settin gs shown in the Sound Blaster manual.

The SB type for Sound Blaster and compatible boards for use

with the "T" parameter is one of the following:

1 - Sound Blaster 1.5 or earlier

Sound Blaster Micro Channel Version

2 - Sound Blaster Pro

3 - Sound Blaster 2.0

Audio Spectrum 16

6 - Sound Blaster 16

8. Interfacing External Equipment

---------------------------------

The primary I/O for Super Morse code is via the computer

keyboard and the PC onboard speaker.  Secondary output is

provided by toggling either the DTR or RTS pin at the selected

RS-232 communications port.  Input is provided via either the DSR

o pin at the same port.  You can select whether to expect either

a high (+) or a low (-) voltage in the key-down condition and

the opposite in the key-up condition. Output via DTR/RTS is

available whenever code is being sent, but input via DSR/CTS is

avail able only in the Manual key function of the Enhance phase

and in the Operate phase.

I/O via either COM1, COM2, COM3, or COM4 can be selected

from the Options|Ports menu selected from the main menu bar. With

a proper interface DTR/RTS can be used to drive an external

oscillator for making high quality code tapes or to key a

transmitt er. Likewise, DSR/CTS can be interfaced with a hand

key, electronic keyer, or a receiver.

Code output to the speaker and COM port can be selected

from the Options menu. Code input can be selected from either one

of several keys on the computer keyboard or the COM port using

the Input Keying selection in the Options|Ports menu.

A detailed explanation of how to use the DTR/RTS and

DSR/CTS signals for the aforementioned purposes is beyond the

Page 13

scope of this manual and is left to your ingenuity.  However,

limited information is provided in Section XI. The author would

like to r eceive diagrams of tested circuits designed by users to

take advantage of COM port I/O.

If an external key does not work, check the following:

a. The RS-232 connector pins are small and easily confused.

Most connectors have the pin numbers beside the pins, but they

are almost impossible to read without a strong light and

magnifying glass. Remember that the connector pins on the

computer ar e a mirror image of those on the cable connector.

b. Some computers connect the RS-232 pins differently.

Consult your owner's manual, or call your computer manufacturer's

technical assistance line for information on the configuration.

Some experimentation (i.e., random poking around in the connecto

r) may be required to find the right pins. On the author's

Gateway 2000, for example, it is necessary to use pin 20 as

signal ground instead of pin 7. This is different from two

previous computers which used pin 7.

c. The pin numbers given in this manual are for the 25 pin

DB-25 connector.  If you have a 9 pin DB-9 connector, match the

pin names instead of numbers.  You will have to consult your

owner's manual for a definition of the pins for that connector.

d. Super Morse uses two COM ports, and you should make sure

that different ports are set for the Modem function and the

keying function.

e. Be sure that the "External" keying source is selected in

the Ports menu.

9. Circuit Diagrams for External Connections

--------------------------------------------

a. Using an External Hand Key

-----------------------------

To RS-232 DSR    o------------+

(Pin 6) or CTS              |

(Pin 5)                     o

/  Morse Telegraph Key

(COMPUTER)                     /   ("Straight" Key)

o

|

To RS-232        o------------+

Pin 7 (*)

GND

Page 14

Page 14

* Due to variations among manufacturers, you may have to

experiment to determine the correct pins

b.  Connecting to a Receiver

----------------------------

No one has submitted a circuit for connection to a radio

receiver, but the following is the way it is done in concept.

+-----------+    +-----------+       +-----------+

DSR/CTS |           |    |           |       |           |

o-------+ Voltage   |    | Tone to   | Audio |           |

| Controlled+<---+ Voltage   +<------+   Radio   |

o-------+ Switch    |    | Converter |       |           |

GND     |           |    |           |       |           |

+-----------+    +-----------+       +-----------+

The connections to the computer are the same as for the

straight key above.  The voltage- controlled-switch can be a

transistor or relay. The tone to voltage converter can be a phase

locked loop, which is available in chip form.  The details are

left to the user, but circuits that can be used or adapted can be

found in the ARRL Handbook and other similar sources.

c. Driving a Transmitter or Other Device

----------------------------------------

The following circuit designed by John Swancara, WA6LOD,

can be used to drive a keyer, code oscillator, or a transmitter.

It is described in QST Magazine, February, 1990, page 36.  A

similar circuit was provided by John Dilks, K2TQN.

If these connections do not work, first check the Port

settings in the Options menu.  Then check your computer's RS-232

information in case you computer uses something different from

the standard.

Page 15

Page 15

+-----------------------+

|  Radio Shack TIL-113  |

|                       |

+-+-+                   +-+-+

|   |

RS-232 Signal   o-----+ 1 +---+    +----------+ 6 +-o NC

ground (Pin 7)              |    |

+-+-+   |    |          +-+-+

|     |    |            |

|     +    |            |

|    \ /   |            |

|     v    |            |

|     -    |            |

|     |    |            |

|     |    |            |

D1     |     | \  |            |

+-+-+   |  \ |          +-+-+

RS-232     R1      /         | \  |

Pin  o--+--\/\/--|< +--+ 2 +---+  \ |      +---+ 5 +-o To

4       |          \              |                Positive

|              +-+-+        |     /    +-+-+   Keying

|                |          |   |/       |     Line

-+-  C1           |          |   |        |

-+-               |          |   |        |

|                |          +---+        |

|                |              |        |

|                |              |        |

|                |              |\       |

o--+                |                \      |

|              +-+-+               _|  +-+-+

|              |   |                   |   |

---        NC o-+ 3 |               +---+ 4 +-+-o To

-              |   |                   |   | |   Trans-

+-+-+                   +-+-+ |   mitter

|                       |   |   Chassis

Board                |                       |   |

Ground               +-----------------------+   |

|

---Interface

- Board

Ground

D1  = 1N914

R1  = 1K Ohm, 1/4 Watt

C1  = 0.01 Microfarad, 50 Volt

DTR = Data Terminal Ready pin

NC  = no connection

Note: (1) Voltage on pins 4 and 5 of TIL-113 should be kept below

20 volts, and the current draw should not exceed 100 ma.

(2) The Philco ECG 3041 may be substituted for the Radio

Shack TIL-113.

Page 16

Page 16

d. Using a Volume-Controlled Headphone

--------------------------------------

Splice a 50 ohm variable resistor and a headphone jack into

the PC speaker line as shown below.  The headphone jack should

break the speaker circuit and close the headphone circuit when

the headphone plug is inserted.

Headphone Jack

o--------+           o     o

|           |     |

\          |     |

PC        50   / <--------+-------------+  /|

Speaker     ohm  \                |       | / |   PC

Line           /                |       |/  |   Speaker

|                 |       |\  |

|                 |       | \ |

o--------+-----------------+-------+  \|

Page 17

Page 17

e. Using an External Oscillator

-------------------------------

Following is a simple code oscillator driven from the COM

port of a PC designed by Jeff Furman, KD6MNP.

DB-25

FEMALE

1N4001

+-----+

|     |       |\ |

|     +-------| \+-------+---+

|  4  |       | /|       |   |

|     |       |/         |   |  555

|     |             +----+---+------+

+-----+             |               |

|     |             |    4   8      |

|     |             |               |

|  7  +------+------+ 1             |

|     |      |      |               |

|     |      |      |               |

+-----+      |      |               |

|      |    6   2   3  |             1/8" stereo

|      +----+---+---+--+             headphone jack

|           |   |   |

|    ||     |   |   |    ||

+----++-----+-+-+   +----++------------+---- V    |

|    ||       |     |    ||            +-- v      |

|             |     |                             |

|              \/\/\                   +----------+

| 0.001 uF       ?     0.01 uF         |                                  film        680k   ceramic or

|               ohms   film            |

|                                      |

+--------------------------------------+

This easily fits inside the DB-25 shell.

Adjust the 680k ohm resistor to change the pitch--

higher value for lower pitch, etc.

Decrease the value of the 0.01 uF cap. for lower volume.

Super Morse Options|Ports settings:

Keying Port  : <your choice from what's available>

Output Keying: ON

RS-232 Output: RTS

Output Sense : -

Page 18

Page 18

f. Using an External Sound Output

---------------------------------

The following was submitted by Bruce G. Kinney, San Jose, CA

+------------------+

30K    Shield|                  |

Pin 20 o--\/\/\/--+-- O + MOD IN           |      /|

|   | |               O  +---- / |

.68uf  -+-  | |               U  |   <   | Speaker

-+-  | |               T  +---- \ |

|   | |                  |      \|

Pin 7  o----------+---+-+ GND              |

|                  |

+------------------+

LEADER 1300S

Function Generator