Pen Services for Microsoft Windows 95

OEM Adaptation Kit

Release Notes

28 February 1995

Table of Contents

Copyright Microsoft Corporation 1995.   All Rights Reserved.

Introduction to the Pen Services for Windows 95 OAK

Pen Services for Windows 95 OAK Contents

Road Map for Pen Services for Windows 95 Adaptation

1. Install the OAK and Build a Sample Installation Diskette

2. Create Drivers for Your Hardware

Microsoft Pen Services for Windows 95 Build Environment

Replacing the Sample Pen Drivers and Driver Configuration Interface

3. Determine Other Changes You Want or Need to Make

Replacing the Windows 95 Control Panel Pen Calibration Library

Replacing the Windows 95 Pen Component Help Files

Replacing the Recognizer

Developing a Windows 95 Pen-Enabled Display Driver

4. Editing the INF File to Reflect Your Changes

Minimal Set of Changes to PENWIN.INF

Editing the PENDRV_Add_Reg .INF File Section

5. Build the Installation Disk Images

6. Test the Pen Services Installation Diskettes You Have Created

Peripheral Vendor Testing

Pen-Capable Windows 95 System Testing

7. Create the Distributed Product

# Introduction to the Pen Services for Windows 95 OAK

Pen Services for Windows 95 is an optional component of the Windows 95 system. It is added to Windows 95 by running the Add/Remove Programs feature of the Windows 95 Control Panel after a Windows 95 system has been installed on a computer. The Add/Remove Programs feature uses Windows 95 Setup to read an information (INF) file from a Pen Services installation diskette supplied by the vendor of a digitizer/Pen Services bundle. The Pen Services for Windows 95 OEM Adaptation Kit (OAK) supplies you with all the information you need to build a Pen Services for Windows 95 installation disk image for your digitizer, as well as the components with which to build a sample installation disk image.

The only source files in this OAK are on-line help source files. Source code for the pen drivers, driver configuration user interface library, and pen calibration library that you need to modify are in the Windows 95 Device Driver Kit (DDK). In order to establish the build environment for modifying these pen drivers and libraries, the Windows 95 Software Development Kit (SDK) must be installed, as well as the Windows 95 DDK.

To test your installation disk image, you will have to have Windows 95 itself installed on an installation test platform.

The Pen Services for Windows 95 installation disk image is distributed in two ways by OEMs:

- OEMs who produce add-on pen tablets for computers running Windows 95 will distribute the installation disk image on a diskette with their tablet. End users who purchase the digitizer tablet for a computer running Windows 95 use the installation diskettes to add Pen Services to Windows 95 when they install the tablet.

- OEMs who produce pen-capable Windows 95 systems use the installation diskettes to create a reference system for adding Pen Services to Windows 95 in the factory. In this case, the installation disk image will be used to create Pen Services installation diskettes to ship with the systems in case reinstallation is ever necessary.

Pen Services for Windows 95 is Version 2.0 of Microsoft Windows for Pen Computing. There are a number of significant differences between Version 2.0 and Version 1.0 of Microsoft Windows for Pen Computing which was shipped with Windows 3.1x. For an overview of these differences, refer to Appendix A of the Programmer’s Guide to Pen Services for Microsoft Windows 95 in the Windows 95 SDK.

From a pen-aware application developer’s perspective, the most significant differences between version 1.0 and 2.0 are in the area of recognition. However, the most important difference for OEMs is in the area of configuration information. In version 1.0, the PENWIN.INI file contained system configuration information; version 2.0 expects to find the configuration information in the Windows 95 registry rather than the PENWIN.INI file. OEMs will have to edit an INF text file (PENWIN.INF) supplied with the OAK so the configuration information that matches their tablet is written into the Windows 95 registry by Setup when the Pen Services installation diskette is used. Details of this change are in the later section “Editing the INF File to Reflect Your Changes.”

# Pen Services for Windows 95 OAK Contents

Note:	See the README.TXT file in the root directory of the CD-ROM for the latest information about this release.

The Pen Services for Windows 95 OEM Adaptation Kit (OAK) includes the following:

Documentation

·	Release Notes (this document),

Software on CD-ROM

·	Pen Services for Windows 95 binary files.

- Sample driver and PENWIN.INF files that can be used to create a sample installation disk image.

·	Pen Services for Windows 95 help source files.

Other Required Software Components

·	Microsoft Windows 95

·	Microsoft Windows 95 Device Driver Kit (DDK)

·	Microsoft Windows 95 Software Development Kit (SDK)

- Microsoft Visual C, version 1.5x (for 16-bit development), Microsoft Visual C, version 2.x (for 32-bit development), and Microsoft Assembler, version 6.11c.

The Microsoft Windows 95 DDK provides sample sources and documentation for the creation of device drivers required by Pen Services for Windows 95. The Windows 95 DDK is available from MSDN.

The files necessary to build Pen Services for Windows 95 applications, such as include files, are in the Windows 95 SDK. The Programmer’s Guide to Pen Services for Microsoft Windows 95 is the documentation that covers topics relevant to building Pen Services for Windows 95 applications. The Microsoft Windows 95 SDK is available from MSDN.

For information on how to obtain a copy of the latest MSDN CD-ROM, contact:

Microsoft Developer Network

One Microsoft Way

Redmond, WA 98052-6399

Fax: (206) 936-7329, Attn: Developer Network

Internet: devnetwrk@microsoft.com

Compuserve: Microsoft Developer Network forum (GO MSDNLIB)

The Microsoft Windows 95 retail release disks are required to test any OEM modifications made to Pen Services for Windows 95. You may obtain Windows 95 disk images for testing purposes only, which in no way constitutes a license to distribute Windows 95. A license agreement with Microsoft is required to ship Windows 95, or any components of  Windows 95. Contact your OEM Account Manager for more information regarding this point if you are unsure as to the scope and nature of your Windows 95 license agreement with Microsoft.

# Road Map for Pen Services for Windows 95 Adaptation

The following steps are required to adapt Windows 95 Pen Services to your hardware:

- Install the OAK (and build a sample installation diskette, if desired).

- Create Windows 95 Pen Services-compatible drivers for your tablet or pen computer.

- Determine other changes (besides replacing the drivers) that you want or need to make to the installation diskette(s) for your product.

- Make the necessary file changes and edit the INF file to reflect the changes.

- Build the installation disk image(s).

- Test the installation diskette(s).

- Distribute the installation diskette(s).

Details about each of these steps form the rest of this document.

## 1. Install the OAK and Build a Sample Installation Diskette

Install the Pen Services for Microsoft Windows 95 OAK by copying it from the CD-ROM distributed by Microsoft to the hard disk on your development machine, using the MS-DOS XCOPY command. Microsoft recommends that you copy the files into a directory named C:\PENOAK and that is the directory name used in examples later in this document. However, the OAK files can be copied into any directory on the hard disk.

Following is an example of using the XCOPY command to install the OAK. If you have the CD-ROM in the E: drive on your computer, then you can install the Pen OAK by using the following form of the XCOPY command from a DOS prompt:

XCOPY  E:\  C:\PENOAK  /S

This causes the following tree to be built on your C: drive:

\PENOAK

|

|----RELNOTES.DOC          // OAK Release Notes (this document)

|

|----\FILES

|         GRECO.DLL      // Microsoft recognizer engine

|         MSKB.DLL       // Microsoft on-screen keyboard library

|         PEN.CNT        // Pen help contents file

|         PEN.HLP        // Pen help file

|         PENC.DRV       // Installable pen driver

|         PENCAL.DLL     // Pen alignment calibration library

|         PENCP.CPL      // Windows 95 control panel pen applet

|         PENUI.DLL      // Pen driver configuration UI

|         PENWIN.DLL     // Pen Services 2.0 API

|         PENWIN.INF     // Pen Services setup script file

|         PENWIN32.DLL   // Pen Services 2.0 API 32-bit thunk layer

|         SK.EXE         // On-screen keyboard application

|         TRAINER.EXE    // Batch trainer application

|         VPENDC.VXD     // Virtual pen driver

|         W_PEN.CNT      // Pen help contents file used by Windows 95

|         WRITEPAL.EXE   // Writing window application

|

|----\HELPSRC

PEN.HPJ

WINDEF.HPJ

PENHELP.H

ACTHAND1.BMP

ACTHAND2.BMP

ACTHAND3.BMP

SMBULL.BMP

CARET.BMP

CHECK.BMP

LASSO.BMP

PENPROC.BMP

PEN_C.BMP

PEN_D.BMP

PEN_H.BMP

PEN_M.BMP

PEN_N.BMP

PEN_P.BMP

PEN_S.BMP

PEN_T.BMP

PEN_U.BMP

PEN_X.BMP

EDITTEXT.RTF

GEST_PR.RTF

INKPROPS.RTF

INSRTTXT.RTF

LENS_LG.RTF

PEN2_PR.RTF

PEN4_PR.RTF

PENPROPS.RTF

PEN_PR.RTF

The \PENOAK\FILES directory contains all the files necessary to build a sample installation diskette. The \PENOAK\HELPSRC directory contains all the source files that were used to build the three help files in the \PENOAK\FILES directory (PEN.HLP, PEN.CNT, and W_PEN.CNT).

To build a sample installation diskette, use the MS-DOS COPY command to copy all the files in the \PENOAK\FILES directory to formatted 3.5” diskette. For example, if the diskette is in the A: drive, at a DOS prompt enter:

COPY  C:\PENOAK\FILES  A:

To test the sample installation diskette, use a computer with Windows 95 installed. Insert the sample installation diskette in a 3.5” driver (the following example assumes the A: drive is the 3.5” drive). Then do the following:

- Start Windows 95.

- Select Settings from the Start menu, and then select Control Panel.

- From among the Control Panel icons, select the Add/Remove Programs icon.

- From the Add/Remove Program Properties sheet, select the Windows Setup tab and then click the Have Disk button.

- When you are prompted to insert the manufacturer’s installation diskette into a drive, insert the sample installation diskette in drive A: and click OK.

- A Have Disk dialog appears displaying the name of the optional component you need to install, Pen Services for Windows 95. Check the box next to the name and click the Install button. Installation will begin.

- When the Add/Remove Program Properties sheet is redisplayed, click OK.

- When you are prompted to restart your computer, click Yes (after taking the sample installation diskette out of the A: drive).

- After Windows 95 restarts. the Pen Driver Configuration Dialog will be displayed. This dialog is generated by the PENUI.DLL component that was installed from the sample installation diskette in the previous steps of this procedure. The drop down list displays the four digitizers supported by the sample tablet driver you just installed. Select the tablet that your computer is using and, if necessary, select the serial device your tablet is using and whether or not the stylus you are using is pressure-sensitive.

- When you are prompted to exit and restart the computer, allow Windows 95 to finish loading properly and then use the Start menu to restart Windows 95. Do not try to use the pen in this intermediate stage because the Pen driver will not be configured until the computer restarts. Use the Start menu to restart Windows 95; do not turn off the computer or use Ctrl+Alt+Del to restart Windows 95. After Windows 95 restarts, the installation of Pen Services for Windows 95 is completed.

Note:     If in step 9 you specify the wrong COM port in the Pen Driver Configuration Dialog, your system 	may reach a state where no input device (pen, mouse, or keyboard) works. If this happens, restart 	Windows 95 and press the F8 key during the restart process to enter Safe Mode. Once Windows 	95 is restarted in Safe Mode, the keyboard (and probably the mouse) will work. Use the keyboard 	or mouse to access the Windows 95 Control Panel Pen icon and then display the Pen Properties 	sheet General tab, where you will have the opportunity to specify a different COM port by using 	the Pen Options button.

Next, you can uninstall this sample installation of Pen Services in preparation for installing the version of Pen Services that supports your hardware.

Note:     Uninstalling Pen Services for Windows 95 does not involve the use of the installation

diskette in any way.

To uninstall Pen Services, perform these steps:

- Select Settings from the Start menu and then select Control Panel. From among the Control Panel icons, select the Add/Delete Programs icon.

- From the Add/Delete Programs Properties sheet, click the Windows Setup tab.

- From the Components list box that is displayed, scroll until you find the entry Pen Services for Windows 95, along with a check box. Uncheck the check box and then click OK.

- When you are prompted to restart your computer, click Yes. After Windows 95 restarts, the sample installation of Pen Services for Windows will be uninstalled.

Now you are ready to begin creating the Pen Services installation diskette that supports your digitizer hardware. This is primarily a matter of replacing files on the sample installation diskette you have just used. The install/uninstall procedures you have just experienced will be the same general procedure users of your installation diskette will employ. Step 8 of the installation procedure will be different because you will change the PENUI.DLL pen configuration dialog to fit your hardware (see the section “Replacing the Sample Pen Drivers and Driver Configuration Interface”).

## 2. Create Drivers for Your Hardware

You must create drivers for your own digitizing device. Refer to the sections “Virtual Pen Device Driver” and “Pen Driver” of the Windows 95 DDK documentation for detailed information on how to do this, although it may as simple as changing a few lines in a header file (as explained in the section “Replacing the Pen Drivers” in this document).

You must also modify the configuration information in the pen configuration dialog presented by the sample PENUI.DLL supplied in the OAK. This dialog must handle your new digitizer drivers’ configuration needs. See the “Pen Driver User Interface” section of the Windows 95 DDK documentation for detailed information on how to do this. An alternative here is to completely remove PENUI.DLL from your installation diskette(s) if you have no user-modifiable configuration settings.

### Microsoft Pen Services for Windows 95 Build Environment

The first step in creating your own drivers is to establish a build environment. In order for you to build binary files that are comparable  with the current shipping versions, you will need to use the same tools that were used to build the shipping versions. The build environment includes a C Compiler, Assembler, Linker, include files and libraries - all of the tools necessary to build the sources provided in this OAK.

If you install both the Windows 95 SDK and Windows 95 DDK on your development system, then you will have all the header files and libraries you need. You also have to install a set of compilers and an assembler.

Follow this sequence of steps in establishing your build environment:

- Install the compilers and assembler. The following are all necessary: Microsoft Visual C, version 2.x (for 32-bit development), Microsoft Visual C, version 1.5x (for 16-bit developement), and Microsoft  Assembler, version 6.11c.

- Next, install the Windows 95 SDK.

- Lastly, install the Windows 95 DDK.

### Replacing the Sample Pen Drivers and Driver Configuration Interface

The sample pen driver (PENC.DRV) and virtual pen driver (VPENDC.VXD) supplied in the OAK support the following digitizers:

- Wacom Technologies Digitizer Model SD-510C

- Wacom Technologies Digitizer Model HD-648A

- Wacom Technologies Digitizer Model UD-0608-R

- The Compaq Concerto digitizer

If your digitizer is compatible with one of the digitizers listed above, then you can build a replacement set of drivers by modifying the DEFINES.H header file supplied with the Windows 95 DDK and using the Windows 95 Pen build environment to build a new set of drivers. For example, suppose your tablet is compatible with the Wacom HD-648A. The DEFINES.H file for pen that is supplied with the Windows 95 DDK has the following lines:

// defines.h -

#ifdef UNIDRV

#define WACOM2     // Wacom SD-510C and Wacom HD-648A

#define WACOM4     // Wacom UD0608-R

#define CPQ        // Compaq Concerto

#endif

#ifdef CPQ

#ifndef BIOS

#define BIOS

#endif

#endif

#define WIN31COMPAT

#define VxD

#define CCODE

#define SysVMIsSpecial

To produce a driver that contains only the code necessary for the Wacom HD-648A, edit the DEFINES.H file to contain only the following lines:

// defines.h -

#ifdef UNIDRV

#define WACOM2     // Wacom SD-510C and Wacom HD-648A

#endif

#define WIN31COMPAT

#define VxD

#define CCODE

#define SysVMIsSpecial

Then recompile and relink the driver.

The pen driver configuration user interface library (PENUI.DLL) is called by the pen driver to provide the user with a dialog box that enables the user to supply hardware-related information about the digitizer to the driver. Examples of such information are the COM port to which the pen tablet is attached, whether or not the pen stylus is pressure-sensitive, etc. An OEM should customize this user interface to present user-adjustable hardware-related options for the particular system in use by modifying the source code in the Windows 95 DDK directory \PEN\SAMPLES\PENUI. For more information on this, see the section “Pen Driver User Interface” in the Windows 95 DDK documentation.

## 3. Determine Other Changes You Want or Need to Make

In the simplest case, you will just replace the pen drivers and pen driver configuration dialog and update the INF file from the sample installation diskette. However, there are additional modifications that an OEM can make to the installation diskette(s):

- Replace the Windows 95 Control Panel pen calibration applet (PENCAL.DLL).

- Replace the Pen help files (PEN.HLP, PEN.CNT, and W_PEN.CNT).

- Replace the handwriting recognizer (GRECO.DLL).

- Edit the Windows 95 setup script file (PENWIN.INF).

One other change you may have to make (in rare circumstances) is to adapt the pen-enabled display driver that is supplied with Windows 95.

The following table gives example reasons for replacing each of the components.

| Component | Example Reason for Replacement | For More Information |
| --- | --- | --- |
| Pen alignment calibration library (PENCAL.DLL) | You may have a digitizer with a resolution other than 640x480 or your digitizer driver may require calibration information that is not in the DRV_CALBSTRUCT structure. | See the “Pen Driver” section of the Windows 95 DDK documentation, where the DRV_CALBSTRUCT structure is defined. |
| Pen help files (PEN.HLP, PEN.CNT, and W_PEN.CNT) | The content of the on-line help files supplied by Microsoft is as hardware-independent as possible and therefore relatively general; you may want to add information specific to your hardware. | See the section “Replacing the Windows 95 Pen Component Help Files” in this document. |
| Recognizer engine (GRECO.DLL) | You may want to create or license an alternate recognizer. | No recognizer source code is provided in the Windows 95 DDK or OAK. See the section “Replacing the Recognizer” in this document. |
| Windows 95 Setup script file (PENWIN.INF) | In the minimal case, you must edit the sample INF file to reflect the changed file sizes of the driver and configuration UI modules. You will most probably also have to edit the hardware initialization values in the INF file. Also, if you change the filenames of any of the files on the sample installation diskette or decide to leave out a module (for example, the recognizer) you will have to edit the file lists in the INF file to reflect those changes. | See the section “Editing the INF File to Reflect Your Changes” in this document. |

### Replacing the Windows 95 Control Panel Pen Calibration Library

The Windows 95 Control Panel pen calibration library (PENCAL.DLL) provides the user with the capability to align the digitizer with the screen as part of running the Control Panel Pen applet. While the pen driver calibration interface attempts to make this interface hardware-independent, you may need to get different calibration information from the user. Your digitizer may have a different resolution than the 640x480 assumed by the sample calibration module. Or, by adding something to the calibration dialog you may be able to achieve optimal resolution for your particular digitizer. Hence, the full source code for the applet is provided as a starting point for further development. The source code is in the Windows 95 DDK directory \PEN\SAMPLES\PENCAL. For more information on this, see the section “Pen Calibration” in the Windows 95 DDK documentation.

### Replacing the Windows 95 Pen Component Help Files

The help file sources for the pen-specific components are provided to allow the OEM to add hardware- dependent items. The pen component Windows 95 Help sources are in the OAK directory \PENOAK\HELPSRC. All the files necessary to build the three distribution help files (PEN.HLP, PEN.CNT, and W_PEN.CNT) are in this directory.

To survey the contents of the help files on the sample installation diskette, either complete the installation of the sample diskette following the procedure given in the section “Install the OAK and Build a Sample Installation Diskette,” or run the Windows 95 WinHelp application on the PEN.HLP file.

When you survey the help topics, if you see topics you want to change, edit the appropriate help topic source file in the OAK directory \PENOAK\HELPSRC. Use Microsoft Word 2.0 or 6.0 to edit the help source files, do not use Windows 95 WordPad. Make sure you do not change the names of any of the help source files.

To rebuild the pen component help files after you change them, use WinHelp 4.0 (the Help Compiler that is in the Windows 95 SDK as part of the Help Development Kit).

The Pen help source files are in the \POAK\HELPSRC directory. The text files that you can edit are .RTF files and there are nine of these. The following table lists the on-line help topics that are contained in each .RTF file.

| File Name | Topics |
| --- | --- |
| EDITTEXT.RTF | Context-sensitive help topics for the Pen Properties sheet displayed by the Windows 95 Control Panel. |
| GEST_PR.RTF | All the on-line help topics under “Becoming a Pen Expert by Using Gestures.” |
| INKPROPS.RTF | Context-sensitive help topics for the Pen Properties sheet displayed by the Windows 95 Control Panel. |
| INSRTTXT.RTF | Context-sensitive help topics for the Pen Properties sheet displayed by the Windows 95 Control Panel. |
| LENS_LG.RTF | Context-sensitive help topics for the Pen Properties sheet displayed by the Windows 95 Control Panel. |
| PEN2_PR.RTF | The following on-line help topic under “Getting Started with a Pen”: Training the Pen to Recognize Your Handwriting. |
| PEN4_PR.RTF | All the on-line help topics under “Tips and Tricks with the Pen: and all the topics under “Troubleshooting the Pen.” |
| PENPROPS.RTF | Context-sensitive help topics for the Pen Properties sheet displayed by the Windows 95 Control Panel. |
| PEN_PR.RTF | All the on-line help topics under the Pen help contents entry “Using a Pen with Text,” all of the topics under “working with Pen Drawings,” plus the following three topics under “Getting Started with a Pen”: Using a Pen as a Pointing Device, Using a Pen for Handwriting, and Using a Pen for Drawing. |

### Replacing the Recognizer

The Microsoft handwriting recognizer (GRECO.DLL) that is included in this OAK may be shipped with your product if you obtain the appropriate licensing agreement from Microsoft. If you do not choose to license the Microsoft recognizer, you can license another vendor’s recognizer or create your own.

For information about developing a handwriting recognizer, see chapters 5 and 8 in the Programmer’s Guide to Pen Services for Microsoft Windows 95 in the Windows 95 SDK.

Developing a Windows 95 Pen-Enabled Display Driver

The Windows 95 standard VGA (640x480) display driver and the Windows 95 standard 8514 display driver are pen-enabled (they are capable of displaying ink as the user draws or enters text with the pen). Also, Pen Services for Windows 95 detects display drivers that use a flat frame buffer. Pen Services is able to perform the ink display function for these type of display drivers so the display driver does not have to be altered in any way to be pen-enabled.

If you must alter a display driver because it supports a resolution unavailable from the Windows 95 VGA and 8514 device drivers and it does not use a flat frame buffer, refer to the Pen section of the Windows 95 DDK for definitions of the functions that have to be added to a display driver for it to display ink.

## 4. Editing the INF File to Reflect Your Changes

An information file (INF file) that Windows 95 Setup can use to install Pen Services for Windows 95 as an optional component of Windows 95 is supplied with the OAK. The name of this file is PENWIN.INF and it can be found in the \PENOAK\FILES directory.

In general, the sample PENWIN.INF file performs the following functions for the Windows 95 system setup program that installs your version of Pen Services for Windows 95:

- Provides the filenames and file sizes of all the files that comprise your version of Pen Services.

- Provides hardware initialization data, which the system setup program puts into the Windows 95 registry.

- Specifies modifications the system setup program must make to the SYSTEM.INI file.

The sample PENWIN.INF file makes a good starting point for creating your PENWIN.INF file, but it is unlikely that the filenames and file sizes or the hardware initialization data in the sample INF file will keep up with the changes you make. The section “Minimal Set of Changes to PENWIN.INF” points out likely areas of change in the PENWIN.INF file. The supplied PENWIN.INF file is well-commented, so look for information there, too.

If your setup procedure goes beyond what you can do by changing PENWIN.INF (for example, if you want to add a custom dialog to the setup procedure or want to decompress files as you copy them to the hard disk, use the InstallShield SE Toolkit provided in the Win32 Software Development Kit to develop your installation program.

### Minimal Set of Changes to PENWIN.INF

An INF file is organized as a set of named sections, much like a Windows .INI file. Each section name is enclosed in square brackets. Following is a list of the sequence of sections that make up the PENWIN.INF file along with the primary use of each section. In order to use the following information, you will have to be looking at a full listing of the PENWIN.INF file that is included in the OAK.

[version]

; You should replace the Provider= string with the name of your company

; instead of Microsoft. Do not change anything else in this section.

[SourceDisksNames]

; Do not change anything in this section as long as all the files on

; your installation diskette fit on one diskette. If your files do not

; fit on one diskette, look at the comments in this section of the sample

; PENWIN.INF file for an example entry that you can add to specify extra

; diskettes.

[SourceDisksFiles]

; This section contains a list of all the filenames on your installation

; diskette and the size of each file. You must change this list to reflect

; any filename changes or file size changes you make. Note that the

; filenames in this section are repeated later in the .INF file in a set

; of “file list” sections that map to destination directories.

[DestinationDirs]

;Do not change any of the following statements in this section:

PenServices.sys             = 11    ;LDID_SYS

Register_PenServices.Copy   = 17    ;LDID_INF

PenDrv_CopyFiles            = 11    ;LDID_SYS

PenDrv_VxD_CopyFiles        = 22    ;LDID_VMM32

If you decide not to ship on-line help for your Pen product, delete the following statement:

PenServices.Help            = 18    ;LDID_HELP

If you decide not to ship any applications, such as the on-screen keyboard, batch trainer, or writing window applications provided by Microsoft, or your own applications, delete the following statement:

PenServices.files           = 25    ;LDID_SHARED

Any other statements in this section are for deleting files installed with Windows 95 Beta releases. In most cases, you can delete these statements. For example, in the Windows 95 final Beta release, the following statements are in the [DestinationDirs] directory:

del.PrevBeta.files          = 25    ;LDID_SHARED

del.PrevBeta.files.sys      = 11    ;LDID_SYS

In most cases, you can delete statements such as this.

[Optional Components]

; Do not change anything in this section.

[Register_PenServices.AddReg]

; Do not change anything in this section.

[penservices]

; Do not change anything in this section.

[penservices_remove]

; Do not change anything in this section.

[PENDRV_Del_Reg]

; Do not change anything in this section.

[PENDRV_Old_Reg]

; Do not change anything in this section.

[PENDRV_Add_Reg]

; This section adds entries to the Windows 95 Registry for every digitizer

; supported by the sample pen driver. In the sample PENWIN.INF file, this

; section registers all four digitizers supported by the sample pen driver.

; All the digitizers registered in this section appear in a

; dropdown list in the pen configuration dialog presented to the user

; during installation of Pen Services. You should edit this section so

; only the digitizer(s) your pen driver supports are registered.

; Note that if your version of the PENWIN.INF file is installing only one tablet

; model, you can suppress the display of the dropdown list completely by editing the

; following line in the [PENDRV_Add_Reg] section:

;   HKLM,System\CurrentControlSet\Services\VxD\VPEND\Model,,” “

To suppress the dropdown list, put the name of your digitizer model in place of the

null string in the above statement. For example, if the only tablet model your

version of PENWIN.INF installs is the Wacom HD-648A, then to suppress the dropdown

list of alternative model numbers, edit the statement as follows:

;   HKLM,System\CurrentControlSet\Services\VxD\VPEND\Model,,”Wacom\HD-648A“

; For more information on how to edit this section, see the topic “Editing the

; PENDRV_Add_Reg .INF File Section” later in these Release Notes.

[PenServices_Add_Reg]

; Do not change this section.

[Register_PenServices.Copy]

; Do not change this section.

[penservices.files]

; This section lists the applications on your installation diskette. The

; sample installation diskette section lists sk.exe (on-screen keyboard),

; trainer.exe (batch trainer), and writepal.exe (writing window application).

; If you change this set of applications on your installation diskette,

; you will have to change this list of filenames. Note that any file named

; in this section must also be listed in the [SourceDiskFiles] section.

[penservices.sys]

; This section lists the files on your installation diskette that are copied

; into the \windows\system directory. Do not change the following entries in

; this list: penwin.dll, penwin32.dll, pencp.cpl, and pencal.dll. If you

; substitute a recognizer for the Microsoft recognizer (greco.dll) or

; do not include a recognizer on your installation diskette, change the

; greco.dll entry accordingly. Note that any file named in this section

; must also be listed in the [SourceDiskFiles] section.

[penservices.help]

; If you decide to include Pen user on-line help in your product, do not

; make any changes to this section. If you decide not to include help with

; your product, delete this section.

[PenDrv_CopyFiles

; Do not change this section.

[PenDrv_VxD_CopyFiles]

; Do not change this section.

[penservices.links]

; This section adds Pen Services items to the Win95 Start:Programs:Pen Services Group

; menu. If you change the set of applications in the [penservices.files] section,

; then you will have to add or delete “setup.ini, group4” entries in this section

; accordingly.

[del.penservices.links]

; This section removes Pen Services items to the Win95 Start:Programs:Pen Services

; Group menu when Pen Services is uninstalled. The entries in this section

; must map to the entries in the [penservices.links] section.

[del.PrevBeta.links]

; This section removes Pen Services items from various Start menus that were

; installed by Windows 95 Beta releases prior to the final Beta. You may want

; to delete this section.

[del.PrevBeta.files]

; This section removes pen applications that were installed by Windows 95 Beta

; releases prior to the final Beta. You may want to delete this section.

[del.PrevBeta.files.sys]

; This section removes pen application support files that were installed by

; Windows 95 Beta releases prior to the final Beta. You may want to delete

; this section.

[penservices.install.reg]

; Do not change this section.

[penservices.remove.reg]

; Do not change this section.

[MS_PENWIN_IniFields]

; Do not change this section. This section adds the entries to the

; drivers= line of the SYSTEM.INI file [Boot] section that are required

; by Pen Services for Windows 95. The [MS_PENWIN_IniFields] section

; of PENWIN.INF appends two fields to the drivers= line: penwindows and

; pen. The drivers= line will look like this:

; [Boot]

...

drivers= ... penwindows  pen

[MS_PENWIN_Ini]

; Do not change either of the “system.ini,drivers” lines of this section. You

; may want to change the level of error reporting parameter in the

; following lines of the section:

; Determines level of non-critical errors flagged

; 0 = show nothing, 1 = show errors, 2 = show errors and warnings

; system.ini,boot,,"penwinerrors=1"

; The two “system.ini,drivers” lines of this section add the following two

; required lines to the [drivers] section of SYSTEM.INI:

;

; penwindows=penwin.dll

; pen=penc.drv”

[NO_MS_PENWIN_IniFields]

; Do not change this section

[NO_MS_PENWIN_Ini]

; If you changed the level of error reporting line in the

; [MS_PENWIN_Ini] section, then you will have to change the comparable

; line in this section.

[Strings]

; This section contains all the translatable strings used in the ,INF

; file. You will have to add a string for your company name and you

; may want to change the Pen_Tip and Tablet_Tip string values.

; All translatable strings are placed in this section

### Editing the PENDRV_Add_Reg .INF File Section

The PENDRV_Add_Reg section of the PENWIN.INF file specifies the hardware-specific digitizer information your device driver needs stored in the Windows 95 registry by the Windows 95 setup program. The sample pen driver needs such information about all four digitizers it supports, so the PENDRV_Add_Reg section of the sample PENWIN.INF file is quite large.

The PENDRV_Add_Reg section in the sample PENWIN.INF is made up of four subsections, one for each digitizer the sample pen driver supports. The minimum number of entries in each of these sample subsections are the following:

- An entry that sets the friendly name of the hardware device that the VxD controls in the Registry HKEY_LOCAL_MACHINE FriendlyName key (see the entry in the sample PENWIN.INF file for the exact path to this Registry key).

- An entry that sets the initial values for the PEN_INFO data structure the VxD uses. This entry sets the HKEY_LOCAL_MACHINE PenInfo key (see the entry in the sample PENWIN.INF file for the exact path to this Registry key).

- An entry that sets the initial values for the HARDWARE_INFO structure the VxD uses. This entry sets the HKEY_LOCAL_MACHINE HardwareInfo key (see the entry in the sample PENWIN.INF file for the exact path to this Registry key).

- An entry that sets the command set that the VxD and digitizer exchanges. This entry sets the HKEY_LOCAL_MACHINE CommandSet key (see the entry in the sample PENWIN.INF file for the exact path to this Registry key).

Some of the digitizer subsections in the sample PENWIN.INF PENDRV_Add_Reg section set other keys and are optional. For example, for the Wacom devices keys are set for the following hardware functionality:

- The initial value of the pressure sensitivity the pen uses to register a point.

- An induction flag for the pen.

- The initial set of rate values.

- The rate command set that the VxD and digitizer exchange.

The PENDRV_Add_Reg section in your PENWIN.INF file will probably support one digitizer, so only the information for one of the digitizers in the sample PENWIN.INF file is used here as a sample, and that is the Wacom 648A tablet.

The following entries are from the sample PENWIN.INF file PENDRV_Add_Reg section for the Wacom 648A. These entries set four different keys under the HKEY_LOCAL_MACHINE\System\CurrentControlSet\Services\VxD key for the sample virtual pen driver (VPEND.VXD) that controls the Wacom 648A.

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,FriendlyName,,"Wacom Technologies Digitizer Model HD-648A"

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,PenInfo,1,6c,20,00,00,60,18,00,00,36,10,00,00,30,0c,00,00,42,00,00,00,00,\

00,00,00,14,00,00,00,c8,00,00,00,01,00,00,00,27,00,00,00,01,00,00,00,02,00,\

00,00,01,00,00,00,46,00,00,00,46,00,00,00,00,00,00,00,00,00,00,00,00,00,00,\

00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,\

00,00,00,00,06,00,00,00

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,CommandSet,1,0d,52,45,0d,00,41,53,31,0d,00,53,52,0d,00,00,00,00,49,43,31,\

0d,00,00,00,00,00,50,48,31,0d,00,00,00,00,00,41,4c,31,0d,00,00,00,00,00,00,\

41,4c,30,0d,00,00,50,48,30,0d,00,53,54,0d,00,53,50,0d,00,52,51,31,0d,00,41,\

4c,31,0d,00,41,4c,30,0d,00

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,Force,1,23,00,00,00

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,Inductive,1,00

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,RateValues,1,14,00,00,00,16,00,00,00,19,00,00,00,1c,00,00,00,21,00,00,00,\

28,00,00,00,32,00,00,00,43,00,00,00,64,00,00,00,c8,00,00,00

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,RateCommands,1,49,54,31,30,0d,00,49,54,39,0d,00,49,54,38,0d,00,49,54,37,0d,\

00,49,54,36,0d,00,49,54,35,0d,00,49,54,34,0d,00,49,54,33,0d,00,49,54,32,0d,\

00,49,54,31,0d,00

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,HardwareInfo,1,01,00,00,00,f8,03,00,00,04,00,00,00,F4,01,00,00,00,00,00,00,\

00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,\

00,00,00,07,00,00,00,03,00,01,00,01,00,15,00,06,00,00,00,37,00,00,00,70,00,\

00,00,00,00,00,00

#### Friendly Name

The FriendlyName key value is the string that is displayed in the pen driver configuration dialog box when the Windows 95 user configures the driver (for example, setting the COM port the digitizer uses).

The FriendlyName entry for the Wacom HD-648A in the sample PENWIN.INF is shown below:

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,FriendlyName,,"Wacom Technologies Digitizer Model HD-648A"

#### PEN_INFO Structure Initial Values

Setup writes the hexadecimal values in the PenInfo entry of the .INF file into the PenInfo key of the registry. When the VxD is loaded and initialized, it reads these values into a PEN_INFO structure.

The PenInfo entry from the sample PENWIN.INF file is shown below:

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,PenInfo,1,6c,20,00,00,60,18,00,00,36,10,00,00,30,0c,00,00,42,00,00,00,00,\

00,00,00,14,00,00,00,c8,00,00,00,01,00,00,00,27,00,00,00,01,00,00,00,02,00,\

00,00,01,00,00,00,46,00,00,00,46,00,00,00,00,00,00,00,00,00,00,00,00,00,00,\

00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,\

00,00,00,00,06,00,00,00

The first part of the entry creates the registry key to hold the initial values for the PEN_INFO structure initial values:

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,PenInfo,1

The value ‘1’ immediately following the key name is not part of the name; instead, it is a code that means the values that follow are hexadecimal values.

Next come the values that are written into the PenInfo key by setup. In order to interpret these values, you need the PEN_INFO structure definition, which is shown below.

typedef struct {

DWORD cxRawWidth;          //physical width of tablet in .0001-inch units

DWORD cyRawHeight;         //physical height of tablet in .0001-inch units

DWORD wDistinctWidth;      //number of distinct values across width of tablet

DWORD wDistinctHeight;     //number of distinct values over height of tablet

DWORD nSamplingRate;       //number of pen packets per second

DWORD nSamplingDist;       //minimum distance between points

DWORD dwSamplingRateMin;   //minimum sampling rate

DWORD dwSamplingRateMax;   //maximum sampling rate

DWORD fuOEM;

DWORD lPdc;                //pen device capabilities

DWORD cPens;               //number of pens supported

DWORD cbOemData;           //width of an OEM data packet

DRV_OEMPENINFO rgoempeninfo[MAXDATAWORDS];

DWORD rgwReserved[8];

} DRV_PENINFO;

The most important thing to notice about the members of this structure are that they are all DWORDs, which are four bytes long. The hexadecimal values in the PENWIN.INF entry are in the same order, or sequence, as the members in the PEN_INFO structure. So, for example, the hexadecimal value 206c is put into the cxRawWidth member of the data structure, x’1860’ is put into the cyRawHeight member, x’1036’ is put into the wDistictWidth member, and so on.

#### Command Set

Following is the entry in the sample PENWIN.INF file for the CommandSet key.

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,CommandSet,1,0d,52,45,0d,00,41,53,31,0d,00,53,52,0d,00,00,00,00,49,43,31,\

0d,00,00,00,00,00,50,48,31,0d,00,00,00,00,00,41,4c,31,0d,00,00,00,00,00,00,\

41,4c,30,0d,00,00,50,48,30,0d,00,53,54,0d,00,53,50,0d,00,52,51,31,0d,00,41,\

4c,31,0d,00,41,4c,30,0d,00

The first part of the entry identifies the Registry key into which Setup should write the rest of the values in the entry as well as specifying that the following values are hexadecimal values:

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,CommandSet,1,

In the hexadecimal values that follow, the value x’0d’ separates each command string from the other. The hexadecimal values in the command strings are ASCII character codes, with null bytes used as filler.

#### HARDWARE_INFO Initial Values

Setup writes the hexadecimal values in the HardwareInfo entry of the INF file into the HardwareInfo key of the registry. When the VxD is loaded and initialized, it reads these values into a HARDWARE_INFO structure.

The HardwareInfo entry from the sample PENWIN.INF file is shown below:

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,HardwareInfo,1,01,00,00,00,f8,03,00,00,04,00,00,00,F4,01,00,00,00,00,00,00,\

00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,00,\

00,00,00,07,00,00,00,03,00,01,00,01,00,15,00,06,00,00,00,37,00,00,00,70,00,\

00,00,00,00,00,00

To interpret this entry, use the same method as is used above to interpret the PenInfo key data. The definition of the HARDWARE_INFO structure can be found in the Pen section of the Windows 95 DDK documentation.

#### Optional Registry Values

The statements in the sample PENWIN.INF file that add optional keys to the registry for the Wacom 648A tablet are shown below as an example of how you can add whatever keys are useful for your digitizer driver. To interpret the values used in these keys, you will have to refer to the Wacom 648A device documentation.

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,Force,1,23,00,00,00

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,Inductive,1,00

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,RateValues,1,14,00,00,00,16,00,00,00,19,00,00,00,1c,00,00,00,21,00,00,00,\

28,00,00,00,32,00,00,00,43,00,00,00,64,00,00,00,c8,00,00,00

HKLM,System\CurrentControlSet\Services\VxD\VPEND\Wacom\HD-648A,RateCommands,1,49,54,31,30,0d,00,49,54,39,0d,00,49,54,38,0d,00,49,54,37,0d,\

00,49,54,36,0d,00,49,54,35,0d,00,49,54,34,0d,00,49,54,33,0d,00,49,54,32,0d,\

00,49,54,31,0d,00

## 5. Build the Installation Disk Images

Build the installation disk image(s) using a combination of executable files provided by Microsoft in the OAK and executable files you have adapted from source code provided by Microsoft or developed yourself.

This disk image can then be copied to formatted diskette(s) which you can distribute as the Pen Services for Windows 95 installation diskette(s). The typical installation will fit on one 3.5” high-density diskette.

## 6. Test the Pen Services Installation Diskettes You Have Created

Methods for testing the installation diskettes differ for peripheral vendors and OEMs who are creating entire pen-capable Windows 95 systems.

### Peripheral Vendor Testing

If you are a peripheral vendor shipping Pen Services for Windows 95 with your peripheral device, then test the installation diskette by using it to install Pen Services on different types of hardware which have Windows 95 already installed. Test the installation under different conditions.

### Pen-Capable Windows 95 System Testing

Install Windows 95 on a reference platform (a system that is typical of the systems you will be distributing). Then use the installation diskette you have created to install Pen Services for Windows 95 on top of Windows 95. This is the first step in the creation of a reference system.

To test the reference system, use the Hardware Compatibility Test (HCT) shipped in the Windows 95 OAK. If passed, the HCT is an excellent indicator that Windows 95 and Windows applications will run as expected with Pen Services for Windows 95 added.

If the reference system has not been severely scaled down for a vertical customer, then the testing process might also include the use of a few major commercial Windows applications to confirm the HCT results. If the reference system has been severely scaled down for a vertical customer, it should be tested in the target environment to ensure that the system meets the needs of the vertical customer.

## 7. Create the Distributed Product

For the peripheral vendor, the Pen Services installation diskettes that have been verified are copied and a copy included in every peripheral unit sold. Instructions on how to use Setup that is included in the installation diskettes should also be included with each unit.

For the pen-capable system vendor, once the reference system has been verified, it will be necessary to clone it to every shipping piece of hardware. Also, a copy of the installation diskette(s) must be included with each system in case the user ever needs to reinstall Pen Services.