SECTION 6	SYSTEM  OPERATION

6.1		SYSTEM BIOS

All microcomputer systems use a Basic Input Output System (BIOS).  This is software that has been permanently recorded in a ROM (Read Only Memory) chip and functions as the basic point of communication between the system board and the rest of the computer.

The BIOS provides an operational interface to the system and relieves the programmer from worrying about the characteristics of hardware devices.  Thus, hardware modifications and enhancements become transparent to user's programs, access to BIOS is through the program interrupts of the 80286 microprocessor.  Each BIOS entry point is available through its own interrupt.

6.2		POWER UP

Upon your turning on the power of your FOX-M286 system, the system will go through a self-test routine which checks all of its internal devices. Complete testings will be carried out on the CPU, base 640K RAM, extended RAM, ROM, system board, CMOS memory, video controllers, parallel and serial subsystems, floppy and fixed disk subsystems and the keyboard.

When the self-test is completed, the system will search for the DOS (disk operating system) system file in drive A.  If no system diskette is put in drive A, it will check the fixed disk (if installed).

6.3		FIRST TIME STARTUP

If it is the first time that the computer is started up, you will need to configure the system by telling the SETUP program what hardware configuration your system contains.

6.4		CONFIGURING YOUR SYSTEM

The AMI BIOS, in addition to the BIOS program itself, contains a setup program that is called up everytime the system boots up.  This is called the SETUP.  The SETUP program lets you specify your system's configuration of diskette drives, hard disk drives, video display, memory, and date and time.  The AMI BIOS provides a one-screen interactive equipment and machine  configuration setup.  The SETUP can be run after the system has been turned on and the memory test is finished or has been escaped.  The SETUP program is built-in, you do not need a diskette to use it.

If your FOX-M286 is already installed in a working system, you will not need to use the SETUP program  unless the configuration already recorded in the on-board CMOS RAM is lost or a change is made in your system hardware configuration.  If the information is lost due to loss of battery power, you will need to reenter the configuration.  If the configuration is altered, the changes must be recorded.

6.4.1 	Memory Test Bypass

After the system is powered on or after a reset, the BIOS performs diagnostics of the system and displays the size of the memory being tested.

Note that you can bypass the memory test by pressing the <ESC> key.  This option would be useful when the memory on the system is quite large.  You may hit the <ESC> key when the message following message appears on the screen:

Press <ESC> Key to bypass MEMORY test

6.4.2	Running Setup

Follow the instructions as they come up on the screen to complete the procedure.  The initial prompt on the screen tells you to press  the <DEL> key if you want to use the setup program.  It is displayed briefly just after the RAM test is run when you first turn on the system.  If it disappears before you have a chance to respond turn the system off and on again or reset the system and the message will reappear.  The initial screen prompt will be similar to:

Press <DEL> key if you want to run SETUP or DIAGS.

Hit <DEL> key to get into the Setup Mode.  (Note that <DEL> key will get you into the setup mode, only when the above message is displayed on the screen.)

Upon your pressing of the <DEL> key, the following message will appear on the screen :

EXIT FOR BOOT

RUN CMOS SETUP

RUN DIAGNOSTICS

Use <Up> and <Down> keys to highlight the selected item.  Highlight RUN CMOS SETUP for the SETUP program and press the <Return> key to enter this option.

In a moment, the following SETUP menu will appear :

Once you have entered the SETUP menu, enter the date, time,  the primary display type, the floppy drive installed and the hard disk drive type (if installed). See Appendix 1 for a printed list of hard disk drive types.  The SETUP program will automatically determines your computer's memory configuration and displays it on the SETUP menu.

Use the <Up>, <Down> , <Left>, <Right>, and <Return> keys to move between options.  The field shown in reverse video is the current field, which is the one the user may changes.  Then use <Pg Up> and <Pg Dn> keys to select the correct values.

After you have entered the correct values to all of the SETUP options, you may now exit the SETUP program.  To do so, press <ESC> key to exit.  The following message will then appear :

Write data into CMOS and exit (Y/N)?

Press Y to update the data and exit the SETUP program.

The computer now performs a cold boot (equivalent to turning the power off and back on again), performs the memory test, and then tries to boot from the disk drive.  If your hard disk has not yet been initialized, be sure that you have a bootable DOS diskette in the A drive.

6.5		RUNNING AMI BIOS DIAGNOSTICS

Following the above procedures until the following display is shown on the screen :

EXIT FOR BOOT

RUN CMOS SETUP

RUN DIAGNOSTICS

Use the <Up> & <Down> to move the highlight bar to select "RUN DIAGNOSTICS" and then press <Enter>. The following screen will then appear :

The AMI Utilities include services for the hard disk, floppy, keyboard, video and miscellaneous. It provides an easy to operate screen-menu allowing inexperienced users to operate the program. Simply press <Left> or <Right> and <Up> or <Down> keys to move the highlight bar the option desired.

SECTION 7	TECHNICAL INFORMATION

This section provides the technical materials about the FOX-M286 system board.  The information in this section is for reference, and is intended for advanced readers who needs to understand the basic design and operation of the FOX-M286 system.

7.1		GC101/GC102 AT CHIP SET

The GC101 Peripheral Controller is the heart of the three chip system and forms most of the control circuits and "glue" logic of the AT architecture in a single CMOS VLSI chip.

The GC101 performs CPU and peripheral support functions including the following:

*	All Peripheral Devices and Refresh Counters

*	All mega functions

-  82284 Clock Generator

-  82288 Bus Controller

-  8254 Timer

-  8259 Interrupt Controller (2)

-  8237 DMA Controller

The GC102 may be configured as either an Address Buffer or Data Buffer by strapping one pin high or low.  This chip replaces address buffer, data transceivers, memory drivers, parity generators and supporting circuitry.

GC102 Data Buffer provides:

*	Buffers and latches data for the CPU expansion bus and memory

*	Parity bit generation and checking

*	Slew rate controlled outputs

GC102 Address Buffer provides:

*	Address buffering for expansion bus, local I/O bus and DRAM address bus.

*	10 bit DRAM address bus

7.2		FOX-M286 SYSTEM BLOCK DIAGRAM

7.3		MICROPROCESSOR

The 80286 is a high-performance microprocessor with a 16-bit external data path, up to 16 megabytes of directly addressable physical memory and up to one gigabyte of virtual memory space.  The operating speed of the 80286 chip is 8 MHz in Normal mode and 16 MHz in Turbo mode.

The 80286 operates in two modes: protected virtual address and real address.

Virtual address mode

The virtual address mode provides a 1-gigabyte virtual address space mapped onto a 16 megabyte physical address space.  Virtual address space is larger than physical address space, and the use of a virtual address that does not map to a physical address location will cause a restartable interrupt.

This mode uses 32-bit pointers that consist of a 16-bit selector and offset components.  The selector specifies an index into a memory-resident table, and the 24 bit base address of the desired segment is obtained from the memory table.  A 16-bit offset is added to the segment base address to form the physical address.  The microprocessor automatically references the tables whenever a segment register is loaded with a selector.  Instructions that load a segment register will refer to the memory-based tables without additional program support.  The memory-based tables contain 8-byte values called descriptors.

Real address mode

In this mode, physical memory is a contiguous array of up to 1 megabyte.  The selector portion of the pointer is interpreted as the upper 16 bits of a 20-bit address, and the remaining 4 bits are set to zero.  This mode of operation is compatible with the 8088 and 8086.

Segments in this mode are 64KB in size and may be read, written or executed.  An interrupt may occur if data operands or instructions attempt to wrap around the end of a segment.  In this mode, the information contained in the segment does not use the full 64KB, and the unused end of the segment may be overlay by another segment to reduce physical memory requirements.

7.4		SYSTEM MEMORY MAP

| Address | Range | Function |
| --- | --- | --- |
| 000000-7FFFFF | 000K-512K | System Board Memory (512K) |
| 080000-09FFFF | 512K-640K | System Board Memory (128K) |
| 0A0000-0BFFFF | 640K-768K | Display Buffer (128K) |
| 0C0000-0DFFFF | 768K-896K | Adaptor ROM (128K) |
| 0E0000-0EFFFF | 896K-960K | System ROM (64K) |
| 0F0000-0FFFFF | 960K-1024K | System BIOS ROM (64K) |
| 100000-7FFFFF | 1024K-8192K | System Memory |

7.5		I/O ADDRESS MAP

I/O Address Map on System Board

I/O address hex 000 to 0FF are reserved for the system board I/O.

| ADDRESS (HEX) | DEVICE |
| --- | --- |
| 000-01F | DMA Controller 1, 8237 |
| 020-03F | Interrupt Controller 1, 8259, Master |
| 040-05F | Timer, 8254 |
| 060-06F | Keyboard Controller |
| 070-07F | Real Time Clock, NMI (non-maskable interrupt) mask |
| 080-09F | DMA Page Register, 74LS612 |
| 0A0-0BF | Interrupt Controller 2, 8259 |
| 0C0-0DF | DMA Controller 2, 8237 |
| 0F0 | Clear Math Coprocessor 80287 Busy |
| 0F1 | Reset Math Coprocessor 80287 |
| 0F8-0FF | Math Coprocessor 80287 |

I/O Address Map on the I/O slots

I/O address hex 100 to 3FF are available on the I/O channel.

| ADDRESS (HEX) | DEVICE |
| --- | --- |
| 1F0-1F8 | Fixed Disk |
| 200-207 | Game I/O |
| 278-27F | Parallel Printer Port 2 |
| 2F8-2FF | Serial Port 2 |
| 300-31F | Prototype Card |
| 360-36F | Reserved |
| 378-37F | Parallel Printer Port 1 |
| 380-38F | SDLC, bisynchronous 2 |
| 3A0-3AF | Bisynchronous 1 |
| 3B0-3BF | Monochrome Display and Printer Adapter |
| 3C0-3CF | Reserved |
| 3D0-3DF | Color Graphics Monitor Adapter |
| 3F0-3F7 | Diskette Controller |
| 3F8-3FF | Serial Port 1 |

7.6		SYSTEM TIMERS

The system has three programmable timer/counters controlled by the timer/counter from the GC101 chip set and are defined as channels 0 through 2 :

| Channel 0 | System Timer |
| --- | --- |
| Gate 0 | Tied on |
| Clk in 0 | 1.190 MHz OSC |
| Clk out 0 | 8259 IRQ 0 |

| Channel 1 | Refresh Request Generator |
| --- | --- |
| Gate 1 | Tied on |
| Clk in 1 | 1.190 MHz OSC |
| Clk out 1 | Request Refresh Cycle |

Note:	Channel 1 is programmed to generate a 15-micro-second period signal.

| Channel 2 | Tone Generation of Speaker |
| --- | --- |
| Gate 2 | Controlled by bit 0 of port hex 61 PPI bit |
| Clk in 2 | 1.190 MHz OSC |
| Clk out 2 | Used to drive the speaker |

The 8254 Timer/Counter is treated by system programs as an arrangement of four programmable external I/O ports.  Three are treated as counters, the fourth is a control register for mode programming.

7.7		SYSTEM INTERRUPTS

Sixteen levels of system interrupts are provided by the 80286 NMI and two 8259 Interrupt Controller within the GC101 chip set.  The following shows the interrupt-level assignments in decreasing priority.

Level					Function

Microprocessor NMI	Parity or I/O Channel

Check

Interrupt Controllers

CTLR 1	CTLR 2

IRQ0				Timer Output 0

IRQ1				Keyboard

(Output Buffer Full)

IRQ2				Interrupt from CTLR 2

IRQ8		Realtime Clock Interrupt

IRQ9		Software Redirected to

INT 0AH (IRQ2)

IRQ10	Reserved

IRQ11	Reserved

IRQ12	Reserved

IRQ13	Coprocessor

IRQ14	Fixed Disk Controller

IRQ15	Reserved

IRQ3				Serial Port 2

IRQ4				Serial Port 1

IRQ5				Parallel Port 2

IRQ6				Diskette Controller

IRQ7				Parallel Port 1

┌──────────────────────────────────────┐

│                                      │

╞══════════════════════════════════════╡

│                                      │

│                                      │

╞══════════════════════════════════════╡

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│     ─┐                               │

│      │ ┌─                            │

│      │ │                             │

│      │ │                             │

│      │ │                             │

│      └─┤                             │

│        │                             │

│        │                             │

│        │                             │

│        │                             │

│        │                             │

│        └─                            │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

└──────────────────────────────────────┘

7.8		DIRECT MEMORY ACCESS (DMA)

The system supports seven DMA channels.  Two DMA Controller are included in the GC101, with four channels for each chip.  The DMA channels are assigned as follows:

| Channel | Function |
| --- | --- |
| 0 | Spare (8 bit transfer) |
| 1 | SDLC (8 bit transfer) |
| 2 | Floppy Disk (8 bit transfer) |
| 3 | Spare (8 bit transfer) |
| 4 | Cascade for DMA Controller 1 |
| 5 | Spare (16 bit transfer) |
| 6 | Spare (16 bit transfer) |
| 7 | Spare (16 bit transfer) |

The following shows the addresses for the page register.

| Page Register | I/O Address (HEX) |
| --- | --- |
| DMA Channel 0 | 0087 |
| DMA Channel 1 | 0083 |
| DMA Channel 2 | 0081 |
| DMA Channel 3 | 0082 |
| DMA Channel 5 | 008B |
| DMA Channel 6 | 0089 |
| DMA Channel 7 | 008A |
| Refresh | 008F |

7.9		REAL TIME CLOCK AND CMOS RAM

The CMOS RAM Chip (146818) contains the real-time clock and 64 bytes of CMOS RAM, it keeps configuration information when power is off.  Upon you turn the system power on, CMOS will load the recorded configuration into the system so that the system can function in the right track with the equipped components.  However, if you haven't configured the CMOS, or the battery which supports the power to the CMOS is weaken, you need to redefine the necessary parameters whenever the system is booting up.

The program of the CMOS setting will be loaded into the system automatically from the sub-system named BIOS.  The following table shows the CMOS RAM addresses.

CMOS RAM Address Map

| Addresses | Description |
| --- | --- |
| 00-0D | * Real-time clock information |
| 0E | * Diagnostic status byte |
| 0F | * Shutdown status byte |
| 10 | Diskette drive type byte - drives A and B |
| 11 | Reserved |
| 12 | Fixed disk type byte - drives C and D |
| 13 | Reserved |
| 14 | Equipment byte |
| 15 | Low base memory byte |
| 16 | High base memory byte |
| 17 | Low expansion meory byte |
| 18 | High expansion memory byte |
| 19-2D | Reserved |
| 2E-2F | 2-byte CMOS checksum |
| 30 | * Low expansion memory byte |
| 31 | * High expansion memory byte |
| 32 | * Date century byte |
| 33 | * Information flags (set during power on) |
| 34-3F | Reserved |

*	These bytes are not included in the checksum calculation and are not part of the configuration record.

Real-time Clock Information

The following table describes real-time clock bytes and specifies their addresses.

| Byte | Function | Address |
| --- | --- | --- |
| 0 | Seconds | 00 |
| 1 | Second alarm | 01 |
| 2 | Minutes | 02 |
| 3 | Minute alarm | 03 |
| 4 | Hours | 04 |
| 5 | Hour alarm | 05 |
| 6 | Day of week | 06 |
| 7 | Date of month | 07 |
| 8 | Month | 08 |
| 9 | Year | 09 |
| 10 | Status Register A | 0A |
| 11 | Status Register B | 0B |
| 12 | Status Register C | 0C |
| 13 | Status Register D | 0D |

Note:	The setup program initializes registers A, B, C, and D when the time and date are set.  Also Interrupt 1A is the BIOS's interface to read/set the time and date.  It initializes the status bytes the same as the setup program.

7.10		MATH COPROCESSOR

The 80287 Math Coprocessor enables the FOX-M286 system to perform high-speed arithmetic, logarithmic functions, and trigonometric operations with extreme accuracy.

The coprocessor works in parallel with the micro-processor.  The parallel operation decreases operating time by allowing the coprocessor to do mathematical calculations while the microprocessor continues to do other functions.

The Mathematics Coprocessor, 80287 is treated as an I/O device through I/O port address hex 0F8, 0FA and 0FC.  The microprocessor sends OP codes and operands through these I/O ports.  The microprocessor also receives and stores results through the same I/O ports.  The "BUSY" signal generated by the coprocessor signifies to the microprocessor to wait until the coprocessor has finished executing.

The following describes the mathematics coprocessor control ports:

0F0		The latched Mathematics Coprocessor busy signal can be cleared with an 8- bit, Out command, to port F0.  The coprocessor will latch "BUSY", if it asserts its error signal.  Data output should be zero.

0F1		The Mathematics Coprocessor will reset to real address mode which is in the 8087 compatible if an 8-bit Out command is sent to port F1. Again, the data output should be zero.

7.11		SYSTEM EXPANSION BUS

The  FOX-M286 provides drive for up to six XT-compatible cards (four of which can be AT-compatible with the second connector)

The I/O channel supports:

*	I/O address space from hex 100 to hex 3FF

*	Selection of data access (either 8 or 16 bit)

*	24 bit memory addresses (16MB)

*	Interrupts

*	DMA channels

*	Refresh of system memory from channel microprocessors

There are six 62-pin (JA1-JA6) and four 36-pin (JB1-JB4) edge connector sockets for I/O channel adapter cards.  In two positions, the 36-pin connector is not present.  These positions can support only 62-pin I/O bus adapters.

The following figure shows the pin numbering for I/O channel connectors JA1 to JA6.

I/O Channel Pin Numbering

(62 Pin)

The following figure shows the pin numbering for I/O channel connectors JB1-JB4.

I/O Channel Pin Numbering

(36 Pin)

The following table summarizes pin assignments for the I/O channel connectors.

I/O Channel (A-Side, JA1-JA6)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| A1 | -I/O CH CK | I |
| A2 | SD7 | I/O |
| A3 | SD6 | I/O |
| A4 | SD5 | I/O |
| A5 | SD4 | I/O |
| A6 | SD3 | I/O |
| A7 | SD2 | I/O |
| A8 | SD1 | I/O |
| A9 | SD0 | I/O |
| A10 | -I/O CH RDY | I |
| A11 | AEN | O |
| A12 | SA19 | I/O |
| A13 | SA18 | I/O |
| A14 | SA17 | I/O |
| A15 | SA16 | I/O |
| A16 | SA15 | I/O |
| A17 | SA14 | I/O |
| A18 | SA13 | I/O |
| A19 | SA12 | I/O |
| A20 | SA11 | I/O |
| A21 | SA10 | I/O |
| A22 | SA9 | I/O |
| A23 | SA8 | I/O |
| A24 | SA7 | I/O |
| A25 | SA6 | I/O |
| A26 | SA5 | I/O |
| A27 | SA4 | I/O |
| A28 | SA3 | I/O |
| A29 | SA2 | I/O |
| A30 | SA1 | I/O |
| A31 | SA0 | I/O |

I/O Channel (B-Side, JA1-JA6)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| B1 | GND | Ground |
| B2 | RESET DRV | I |
| B3 | +5 Vdc | Power |
| B4 | IRQ9 | I |
| B5 | -5 Vdc | Power |
| B6 | DRQ2 | I |
| B7 | -12 Vdc | Power |
| B8 | 0WS | I |
| B9 | +12 Vdc | Power |
| B10 | GND | Ground |
| B11 | -SMEMW | O |
| B12 | -SMEMR | O |
| B13 | -IOW | I/O |
| B14 | -IOR | I/O |
| B15 | -DACK3 | I |
| B16 | DRQ3 | O |
| B17 | -DACK1 | I |
| B18 | DRQ1 | O |
| B19 | -Refresh | I/O |
| B20 | CLK | O |
| B21 | IRQ7 | I |
| B22 | IRQ6 | I |
| B23 | IRQ5 | I |
| B24 | IRQ4 | I |
| B25 | IRQ3 | I |
| B26 | -DACK2 | O |
| B27 | T/C | O |
| B28 | BALE | O |
| B29 | +5 Vdc | Power |
| B30 | OSC | O |
| B31 | GND | GROUND |

I/O Channel (C-Side JB1-JB4)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| C1 | SBHE | I/O |
| C2 | LA23 | I/O |
| C3 | LA22 | I/O |
| C4 | LA21 | I/O |
| C5 | LA20 | I/O |
| C6 | LA19 | I/O |
| C7 | LA18 | I/O |
| C8 | LA17 | I/O |
| C9 | -MEMR | I/O |
| C10 | -MEMW | I/O |
| C11 | SD8 | I/O |
| C12 | SD9 | I/O |
| C13 | SD10 | I/O |
| C14 | SD11 | I/O |
| C15 | SD12 | I/O |
| C16 | SD13 | I/O |
| C17 | SD14 | I/O |
| C18 | SD15 | I/O |

I/O Channel (D-Side JB1-JB4)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| D1 | -MEM CS16 | I |
| D2 | -I/O CS16 | I |
| D3 | IRQ10 | I |
| D4 | IRQ11 | I |
| D5 | IRQ12 | I |
| D6 | IRQ15 | I |
| D7 | IRQ14 | I |
| D8 | -DACK0 | O |
| D9 | DRQ0 | I |
| D10 | -DACK5 | O |
| D11 | DRQ5 | I |
| D12 | -DACK6 | O |
| D13 | DRQ6 | I |
| D14 | -DACK7 | O |
| D15 | DRQ7 | I |
| D16 | +5 Vdc | Power |
| D17 | -MASTER | I |
| D18 | GND | Ground |

APPENDIX

FOX-M286 SYSTEM BOARD LAYOUT

Hard Disk Drives supported by AMI BIOS

| Type | Cylinders | Head | Write Precomp | Landing Zone | Capacity |
| --- | --- | --- | --- | --- | --- |
| 1 | 306 | 4 | 128 | 305 | 10MB |
| 2 | 615 | 4 | 300 | 615 | 21MB |
| 3 | 615 | 6 | 300 | 615 | 31MB |
| 4 | 940 | 8 | 512 | 940 | 64MB |
| 5 | 940 | 6 | 512 | 940 | 48MB |
| 6 | 615 | 4 | None | 615 | 21MB |
| 7 | 462 | 8 | 256 | 511 | 31MB |
| 8 | 733 | 5 | None | 733 | 31MB |
| 9 | 900 | 15 | None | 901 | 115MB |
| 10 | 820 | 3 | None | 802 | 21MB |
| 11 | 855 | 5 | None | 855 | 36MB |
| 12 | 855 | 7 | None | 855 | 51MB |
| 13 | 306 | 8 | 128 | 319 | 21MB |
| 14 | 733 | 7 | None | 733 | 44MB |
| 15 | Reserved |  |  |  |  |
| 16 | 612 | 4 | All | 663 | 21MB |
| 17 | 997 | 5 | 300 | 997 | 42MB |
| 18 | 997 | 7 | None | 997 | 58MB |
| 19 | 1024 | 7 | 512 | 1023 | 61MB |
| 20 | 733 | 5 | 300 | 977 | 31MB |
| 21 | 733 | 7 | 300 | 732 | 42MB |
| 22 | 733 | 5 | 300 | 733 | 31MB |
| 23 | 306 | 4 | All | 336 | 10MB |
| 24 | 925 | 7 | All | 925 | 56MB |
| 25 | 925 | 9 | None | 925 | 72MB |
| 26 | 754 | 7 | 754 | 754 | 46MB |
| 27 | 754 | 11 | None | 754 | 72MB |
| 28 | 699 | 7 | 256 | 699 | 46MB |
| 29 | 923 | 10 | None | 823 | 71MB |
| 30 | 918 | 7 | 918 | 918 | 55MB |
| 31 | 1024 | 111 | None | 1024 | 98MB |
| 32 | 1024 | 15 | None | 1024 | 133MB |
| 33 | 1024 | 5 | 1024 | 1024 | 44MB |
| 34 | 624 | 2 | 128 | 612 | 10MB |
| 35 | 1024 | 9 | None | 1024 | 80MB |
| 36 | 1024 | 8 | 512 | 1024 | 71MB |
| 37 | 615 | 8 | 128 | 615 | 42MB |
| 38 | 987 | 3 | 987 | 987 | 25MB |
| 39 | 987 | 7 | 987 | 987 | 60MB |
| 40 | 820 | 6 | 820 | 820 | 42MB |
| 41 | 977 | 5 | 977 | 977 | 42MB |
| 42 | 981 | 5 | 981 | 981 | 42MB |
| 43 | 830 | 7 | 512 | 830 | 50MB |
| 44 | 830 | 10 | None | 830 | 72MB |
| 45 | 917 | 15 | None | 918 | 115MB |
| 46 | 1224 | 15 | None | 1223 | 152MB |
| 47 | USER TYPE |  |  |  |  |

SUMMARY OF SYSTEM BOARD SETTINGS

| Switch 1 | Function |
| --- | --- |
| ON | Zero wait state |
| OFF | One wait state |

| Switch 2 | Function |
| --- | --- |
| ON | 256KB DIPs |
| OFF | 1M SIMM |

| Switch | Setting |  |  |
| --- | --- | --- | --- |
| 3 | 4 |  | Memory configuration |
| Using 4 | 4256K/1M | SIMM: |  |
| SW 2 | SW 3 | SW 4 |  |
| ON | ON | ON | 256KB / 0KB |
| ON | OFF | ON | 512KB / 0KB |
| ON | ON | OFF | 640KB / 0KB |
| ON | OFF | OFF | 640KB / 384KB |
| OFF | ON | OFF | 640KB / 1408KB |
| OFF | OFF | OFF | 640KB / 3456KB |

| Jumper | Pin | Function |
| --- | --- | --- |
| JP1 | 1,2 | Parity bit disable |
|  | 2,3 | Parity bit enable |
| JP2 | 1,2 | 256K ROM BIOS |
|  | 2,3 | 128K ROM BIOS |
| JP3 | short | CGA / EGA / VGA |
|  | open | Monochrome |
| JP4 | 2,3 (default) | DRAM speed selector |

| Connector | Function |
| --- | --- |
| P1 | Speaker Connector |
| P2 | Hardware Reset Connector |
| P3 | Turbo LED Connector |
| P4 | Power LED & Ext-Lock Connector |
| P5 | Turbo Switch Connector |
| P6, P7 | Power Supply Connector |
| P8 | External Battery Connector |
| KB1 | Keyboard Connector |