%OP%DESystem Calls Reference

%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%CO:A,5,74%GN_PMD, put (set) the current machine date

RST 20H, DEFW $1C09

IN:

OUT, if call successful:

Registers changed after return:

Notes:

Bugs:

Related calls:

GN_DEI, convert zoned format date to internal format

GN_DIE, convert internal format date to zoned format

GN_GDT, convert an ASCII string to an internal binary date

GN_GMD, Get (read) machine date in internal format

GN_PDT, write internal date as ASCII string

%P0%

%CO:B,5,69%

HL = 0, not allowed

HL = 1, not allowed

HL = 2, 3 byte internal date in ABC (A = high byte, C = low byte)

HL > 255, HL points to 3 byte internal date (low byte first)

-

AFBCDEHL/IXIY same

......../.... different

The date is used throughout the machine and should only be altered

if the user of the machine is fully aware of the fact.

Flow sequense:

1. Get real time in days/seconds/centiseconds

2. Reset RTC

3. Multiply set-days by $011580

4. Add real seconds.

5. Save new base seconds

%CO:C,5,64%%CO:D,5,59%%CO:E,5,54%%CO:F,5,49%